Air Liquefaction and Cryogenics – Report 1 (2021)

LOX Prototype

Author: Mariam El Rez

Editor: Samir Mourad

Last update: 16 September 2021

Contents

1.	Portotype	4

1.1.	Air Compressor	4

1.2.	Prototype cycle of Oxygen liquefaction	6

1.3.	Prototype Heat exchanger (HX - N2/N2)	7

1.4.	Prototype Heat exchanger (HX - N2/O2)	10

2.	Components of Oxygen liquefaction prototype:	14

3.	Heat exchangers for prototype project:	15

3.1. HX- N2/N2	16

3.2.	HX-N2/O2 main	18

3.3.	HX-N2/O2 (2nd)	19

3.4.	HX- final calculation	21

4.	FreeCad Design	22

4.1.	Prototype design on FreeCad	22

4.2.	Heat exchanger design HX-N2/N2	23

4.3.	Heat exchanger design HX-N2/O2 [Main]	26

4.4.	Heat exchanger design HX-N2/O2 [2nd]	28

4.5.	Cooling design [ Inside kelvinator ]	31

4.6.	Cooling FreeCad design	31

5.	Price of prototype components	37

6.	Real design of prototype	38

7.	Real design of cooling (inside kelvinator refrigerator)	45

8.	First experiment (Expr #1)	46

9.	P&I diagram	49

A-	P&I diagram without heat exchangers (Expr. #1)	49

B-	P&I diagram with heat exchangers (Expr. #2)	49

10.	Prototype installation (Expr.#1)	50

## Portotype

## Air Compressor

#1 (with oil)

#2 (oil free)

Air Compressor (Antar store)

- 10 bar , 50L price 165 dollars (makute company)

- 10 bar, 150L price 395 dollars

- 10 bar, 200 L price 450 dollars

## Prototype cycle of Oxygen liquefaction

In this prototype the oxygen will be liquefied by cascade cooling of nitrogen.

The nitrogen gas will be compressed (from 2 bar to about 15 bar)[use for that the laboratory refrigerator], The nitrogen will then be cooled down to 195 K by means of a Kelvinator fridge operated with a cascade of R-502 and R-503 refrigerants.

Then the nitrogen will be cooled to lower temperatures (83.6 K) using the expansion valve and heat exchanger.

This nitrogen temperature (<90 K) would be sufficient to liquefy the oxygen at 1atm.

Oxygen gas can also be prepared and cooled to about 170 K in nitrogen before returning directly to the compressor (160 K).

  - Prototype Heat exchanger (HX - N2/N2)

LMTD Correction Factor Charts

Calculates Logarithmic Mean Temperature Difference (LMTD) Correction factor for different configuration of exchangers.

  - Data:

  - Result:

Shortcut Heat Exchanger Sizing

Estimates LMTD (Log Mean Temperature Difference), Exchanger surface area, number of tubes, shell diameter and number of shell in series.

  - Data:

  - Result:

  - Prototype Heat exchanger (HX - N2/O2)

LMTD Correction Factor Charts

Calculates Logarithmic Mean Temperature Difference (LMTD) Correction factor for different configuration of exchangers.

  - Data:

  - Result:

Shortcut Heat Exchanger Sizing

Estimates LMTD (Log Mean Temperature Difference), Exchanger surface area, number of tubes, shell diameter and number of shell in series.

  - Data:

  - Result:

- Components of Oxygen liquefaction prototype:

- Compressor: It is LR25B Laboratory refrigerator compressor

- Cooling of N2: using kelvinator refrigerator

- Heat exchanger (HX-N2 / N2): should be manufactured

- Heat exchanger (HX-N2 / O2): should be manufactured

- Cooling of O2: should be manufactured

- Expansion valve: Purchase

- Filter drier: Purchase

- Cryometer (measures up to 80 K): Purchase

- Connections: The available LR25B Laboratory refrigerator and air-conditioner parts can be used if they are suitable for work.

- Gaseous oxygen preservation tank: manufacture / purchase

- Liquid oxygen storage tank: manufacture / purchase

- Gaseous oxygen (volume?)

- Nitrogen (volume?)

- Thermal isolations

*Cryometer

- Features

Digitized the conventional type (MBM) to a more compact design.

- In combination with the MBS CRYO-METER, the accurate temperature can be obsrved at remote position.

- MBD CRYO-METER can be used as a power sorce unit of the MBS CRYO-METER.

- Specification

- Heat exchangers for prototype project:

The central variables in any heat exchanger analysis are the heat transfer rate q [W], heat transfer area A [m2], heat capacity rates C& (=m& cp) [W/K], and the overall heat transfer coefficient U. On the basis of these variables and the fluid temperatures, we can write two basic equations for the heat transfer rate; first, for heat transfer rate it must hold that

q = U A Tm

where Tm is the average (mean) temperature difference of the two fluids in the heat exchanger, and the area A in equation the heat transfer area, meaning the contact area between one of the fluids, and the surface of the wall that separates the fluid.

Second, on the basis of 1st law of thermodynamics, the heat transfer rate q must also equal the rate of heat lost by the hot fluid stream and gained by the cold fluid stream:

## 3.1. HX- N2/N2

## HX-N2/O2 main

  - HX-N2/O2 (2nd)

  - HX- final calculation

- FreeCad Design

  - Prototype design on FreeCad

## Heat exchanger design HX-N2/N2

6 shell , 12 tubes

Shell – Cold side – N2

Tubes – Hot side – N2

## Heat exchanger design HX-N2/O2 [Main]

1 shell , 2 tubes

Shell – Cold side – N2

Tubes – Hot side – O2

## Heat exchanger design HX-N2/O2 [2nd]

1 shell , 8 tubes

Shell – Cold side – N2

Tubes – Hot side – O2

## Cooling design [ Inside kelvinator ]

Qconv  =  m dot * Cp* (Ti - To)

=  25 kg/h *  1.088375 KJ/Kg.K * (335 - 195)

= 25/3600 * 1.088375 *140 = 1.0575 KW = 1057.5 W

Qconv = h*As*LMTD    or    LMTD = ΔTo- ΔTiln⁡(ΔToΔTi) = 33.2

As =   1057.5h*33.2 = 1057.525*33.2 = 1.274 m2

Or  As = 2 * π * R * h   → h  = As2*π*R = 1.274π*9.525*10^-3 = 42.58 m

## Cooling FreeCad design

- Price of prototype components

- General space of prototype is  : L * W * H = 1.93 m * 2 m * 1.2 m (except height of kelvinator refrigerator = ~ 1.7 m)

- Real design of prototype

- Real design of cooling (inside kelvinator refrigerator)

In this evaporator, we have 20 pipes (size 3/8, length L=~ 32 cm) with 19 tees ( Radius R= 1.5 cm)

So: (32cm x 20 pipes)+( 2 x π x 1.5cm x 180˚ 360 x 19 tees) = 729.535 cm = ~ 7.29 m

The length required for cooling was previously calculated : 42.58 m =~43 m

43 m /7.29 m = 5.98 floor  → we need 6 floor for cooling

But the measurements of evaporator are not accurate so we will use 7 floors or pieces of this evaporator

Price of one piece of evaporator = ~200 000 L.L.

Total price excluding hand cost = ~ 200 000 x 7 = ~ 1 400 000 L.L.

- First experiment (Expr #1)

The first experiment (Expr #1) aims to:

1- Compressor operation test with nitrogen gas instead of R-134a,

2- Make sure that the Kelvinator refrigerator is running correctly

3- Ensure that the expansion valve is compatible with the design.

Therefore, the three heat exchangers (HX-N2/N2, HX-N2/O2 main, HX-N2/O2 2nd) will be excluded from this experiment.

In this experiment, the oxygen will not be liquefied, but only the components will be tested and designed to ensure the proper functioning of the refrigeration cycle.

The first experiment consists of a simple cycle consisting of a compressor (LR25B Laboratory) with a condenser, cooling through a Kelvinator refrigerator, and an expansion valve (taken from an LR25B Laboratory refrigerator).

The components that must be provided to carry out this experiment:

    - Filling the design with nitrogen gas, immediately before the pump.

    - Covering the design with a thermal insulation material to maintain the temperature of the refrigeration cycle,

    - The presence of a thermometer (below -100˚C (173 K)) to measure the cooling outlet (inlet of the expansion valve) and the outlet of the expansion valve,

    - Also, a weather thermometer (from -10˚C to 20˚C (263 K to 293 K)) to measure the temperature of the compressor inlet.

For the safety of the compressor:

- In the first hour of operation, a 5-minute break must be taken for the compressor every 15 minutes of operation, in order to avoid an explosion of the compressor or one of the designed cables.

- Also, the compressor must not run for more than two hours in a row.

- Also, cold nitrogen gas must be entered into the compressor to reduce the speed of nitrogen flow, in addition to its role in cooling the compressor.

During this experiment (Expr. #1), the amount of nitrogen gas filled to the cycle, the time of the experiment, the temperature reached by the refrigeration cycle, the pressure during operation will be calculated.

- P&T diagram

  - P&T diagram without heat exchangers (Expr. #1)

  - P&T diagram with heat exchangers (Expr. #2)

- Prototype installation (Expr.#1)

Literature

…

http://calc.exergyllc.com/

https://checalc.com/calc/ShortExch.html