MEGBI - Middle East Genetics and Biotechnology Institute

مركز أبحاث للجينات والتقنية البيولوجية

MEGBI-APP (Antibiotics Pilot Plant) Report 2023

- Penicillin and Ampicillin production and quantification (Lab scale)

- Aspirin production and quantification (Lab scale and Pilot Plant Scale)

- Generic Pilot Plant Design for Penicillin/Ampicillin/Aspirin Pilot Plant

Authors:

Mohammad Kalawoun

Abdullah Mourad

Ghina Abdel-Hamid

Rama Uthman

Ghinwa Younes

Cynthia Baissary

Salwa Othman

Last Update: 30.12.2023 14:14

Edited on 7.4.2025 by Dr. Samir Mourad

Table of Contents

Preface	1

1	Part I: Aspirin Lab Scale Production and Qualification	2

1.1	Aspirin Qualification	2

1.2	Finalized Price List	3

1.3	Aspirin Optimization	5

1.3.1	Introduction	5

1.4	Results	6

1.5	Discussion	9

1.6	Conclusive Remark	10

2	Part II: Penicillin Lab Scale Production and Quantification	11

2.1	Penicillin Production and Quantification May2023	11

2.1.1	Penicillin Quantification (May 2023)	11

2.2	Results	18

2.2.1	Results	22

3	Part III: Ampicillin Lab scale Production	26

3.1	Ampicillin Production and Quantification 2023	26

4	Part IV: Generic Pilot Plant Design (Penicillin/Ampicillin/Aspirin)	41

4.1	Mechanical design	41

4.1.1	MEGBI penicillin/ampicillin/asprin pilot scale production old design	41

4.1.2	update for pilot plant ASPIRIN/PENICILLIN production 18/11/2023	42

4.1.3	update for pilot plant ASPIRIN/PENICILLIN production 23/11/2023	43

4.1.4	MEGBI penicillin/ampicillin/aspirin pilot scale production FreeCAD design 28/11/2022	44

4.1.5	MEGBI Penicillin/ampicillin/aspirin pilot scale production FreeCAD design 16/12/2022	44

4.1.6	MEGBI Penicillin/Ampicillin/Aspirin pilot scale production FreeCAD design 23/01/2023 (achieved)	45

4.1.7	MEGBI Aspirin filter and crystallizer pilot scale FreeCAD design 09/01/2023	45

4.1.8	Sizes of filter/crystallizer aspirin pilot plan	46

5	Part V: Aspirin Pilot Plant	47

5.1	Requirements For Aspirin Pilot Plant Production	47

5.2	Pilot plant system design	49

5.2.1	Dynamic View of Aspirin Pilot Plant Production	49

5.3	Aspirin pilot plant Mechanical Realization	54

5.3.1	Stand installation (18/01/2023-07/04/2023)	54

5.4	Test specification	108

5.4.1	Autoclave System Test Specification	108

5.4.2	Safety precaution	108

5.4.3	Autoclave operation sequence	108

5.4.4	Autoclave System testcases	108

5.4.5	Aspirin Production System Test Specification	110

5.4.6	Aspirin Production operation sequence	110

5.4.7	Aspirin Production System testcases	110

5.5	System tests	115

5.5.1	003: Aspirin Production System Test (Water test)	115

5.5.2	004: Aspirin Production System Test (Water test)	123

5.5.3	005: Aspirin Pilot Plant flow sensor Test	131

5.5.4	006: Aspirin Production System Test (Water test)	137

5.5.5	007: Aspirin Pilot Plant flow sensor Test	145

5.5.6	008: ASPIRIN PILOT PLANT TEST (AUTOCLAVE TEST)	147

5.6	Cleaning/Tableting/Recycling Pilot plant	151

5.6.1	Cleaning process	151

5.6.2	Aspirin tableting process	155

5.6.3	Evaluation test	159

5.6.4	Recycling	161

6	Annex: The most important solutions in biology lab	165

6.1	Making bacterial stock solution:27-10-2022	165

6.2	Realisation:28-10-2022	165

6.2.1	Solution preparation	166

6.3	Preparation of the turbidity calibration 0.5 McFarland:(40)	168

6.4	Prices of laboratory products:	169

6.5	MEGBI-Suppliers contact:	169

References	170

Preface

The pharmaceutical industry is a significant pillar of any healthcare system, and its scope covers drug discovery, development, manufacturing, and marketing. In light of the COVID-19 outbreak, medicine accessibility was challenged worldwide, putting many people’s lives at risk. High levels of drug consumption and insufficient local pharmaceutical manufacturing remain a big challenge for the pharmaceutical supply chain and healthcare system. Localizing the manufacturing of drugs, and their ingredients in residence country, is vital to protect any country’s healthcare system and enhance its readiness for emerging outbreaks beyond COVID-19 like several others sudden outbreaks which could spread at any time.

Local production of pharmaceuticals plays a vital role in maintaining resilience of national healthcare systems, especially when it comes to facilitating access to needed medicines and decreasing exposure to imports and international supply chains. Pharmaceutical companies typically operate in both national and international markets, through which they are subjected to specific regulations and healthcare policies that govern drug manufacturing, approval, marketing, and sales. These legislations are different from one country to another, depending on the healthcare challenges they face, and could directly influence the discovery, development, manufacturing, and sales of new drugs.

In any world countries generally, and in Lebanon specially ,drugs are the main product needed by the consummer to treat even to prevent several diseaes. And the main type of drug to be used is antibiotics (penicillin, ampicillin...), even Aspirin which is the most widely sold over-the-counter drug.

For Lebanon, there are not enough studies even local industries to produce even the basic drugs or medecines like these mentionned above, for this reason the import still the ideal solution to secures the locally need. According that, we as AECENAR, and specially as MEGBI institute, we try to find and even to produce these medecines locally by developping a bioreactor which can produce this types of drugs.

# Part I: Aspirin Lab Scale Production and Qualification

## Aspirin Qualification

Here we can find the price list of chemicals needed for testing:

| Aspirin quality control standard tests according to the Ph.Eur. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Reagent | Quantity per experiment | Quantity ordered | Price | Reference | Missing Apparatus |  |
| IR absorption | Potassium bromide | 300-400 mg |  |  |  | IR Spectrophotometer, hydraulic press |  |
| Color test | Calcium hydroxide | 0.5 g | 500 g | $63.53 | Biosynth |  |  |
| Color test | 2-Nitrobenzaldehyde solution | 0.05 mL | 0.25 kg | $50.82 | Biosynth |  |  |
| Appearance of solution | Reference solution B9 | For comparision | 100 mL | $107.70 | Reagecon |  |  |
| Related substances | Phosphoric acid |  |  |  |  | HPLC |  |
| Related substances | Acetonitrile |  |  |  |  | HPLC |  |
| Heavy metals | Thioacetamide solution | 1.20 mL | 25 g (19.7 mL) | $69.00 | Sigma |  |  |
| Heavy metals | Buffer solution | 2 mL | 1 L | $63.49 | Reagecon |  |  |
| Heavy metals | Lead standard solution | 10 mL of 1 ppm | 100 mL of 100 ppm | $127.42 | Reagecon |  | Note: needs to be in water |
| Loss on drying | Diphosphorus pentoxide | Amount depends on size of desiccator | 500 g | $154.00 | Sigma | vacuum desiccator, weighing bottle |  |
| Sulphated ash | Silica gel |  |  |  |  | crucible, muffle furnace, desiccator |  |
| Assay | Phenolphthalein | As an indicator | 50 g | $44.84 | VWR |  |  |
| Total |  |  |  | $680.80 |  |  |  |

## Finalized Price List

In order to work according to priorities, the first involves ordering large amounts of salicylic acid and acetic anhydride. As for the aspirin qualification, due to the specific apparatus that is required, we have decided to move forward with conducting a melting point test and therefore purchasing that apparatus. As for the remaining tests, those will be done at specialized laboratories, such as the university labs listed by the MOPH in order to register and certify a drug. In addition, this table includes components that are missing from the lab but are vital to the projects we are conducting, both now and in the future. Finally, we have also included the reagents required for phenylacetic acid production.

| Title | Reagent | Quantity ordered | Price | Reference | Total |
| --- | --- | --- | --- | --- | --- |
| Aspirin production | Acetic anhydride | 5 L | $150.00 | VTC (VWR)/in stock | $150.00 |
| Lab Equipment | Separatory funnel (1000 mL) |  | $32.00 | VTC | $713.00 |
|  | Complete Buchner filtration apparatus (1000 mL) |  | $150.00 | VTC |  |
|  | Stand |  | $16.00 | VTC |  |
|  | Capillary tubes | 100 units | $5.00 | VTC |  |
|  | Separatory funnel (500 mL) |  | $20.00 | City Med Lab |  |
|  | Safety goggles | 3 units | $18.00 | City Med Lab |  |
|  | Fire Extinguisher | 2 kg | $12.00 |  |  |
|  | Distillation (Vacuum) |  | $90.00 | Amazon |  |
|  | Shipping: |  | $40.00 |  |  |
|  | Melting point apparatus |  | $200.00 | Alibaba |  |
|  | Shipping: |  | $130.00 |  |  |
| PAA production | Methyphenyl acetate | 1 kg | $210.00 | VTC | $357.00 |
|  | Sodium hydroxide (2M) | 1 kg | $5.00 |  |  |
|  | Ether | 2.5 L | $125.00 | VTC |  |
|  | Na2SO4 | 1 kg | $17.00 | VTC |  |
| Total |  |  | $1,220.00 |  | $1,220.00 |

## Aspirin Optimization

### Introduction

With the arrival of the melting point apparatus, the sample that had originally been produced on 25/11/2022 during the aspirin production protocol was tested for its melting point. Its range was approximately 114-116oC. In light of the impurities that have lowered the melting point, trials were conducted varying different parameters to both test the effect these parameters have on yield as well as on purity.

## Results

| Trial | Volume | Molarity of Acid | Recrystallization solvent | Reaction time | Mass of Product | % Yield | FeCl3 test | Melting Point |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 mL acetic anhydride | 1M (sulfuric acid) | warm water | 10 min 10min 10min 15min 15min | 1.96g - - 1.56g 1.21g | 75.1% 60% 46.3% | + + + + + | 120.6-121.4 °C 121.5-124.0 °C 120.8-124 °C 125-127 °C 126.7-130.5 °C |
| 2 | 5 mL acetic anhydride | 6M | water | 10 min | 1.60g 2.42g 1.07g | 61.3% 92.7% 41.15% | + + + | 117.7-119.6 °C 121-128 °C 123.7-125.8 °C |
| 3 | 5 mL acetic anhydride | 6M | ethanol and water ethanol and cold water ethanol and warm water ethanol and warm water ethanol and warm water ethanol (10ml) and warm water ethanol 5%, distilled water at room temperature ethanol (10ml) and warm water | 10 min 15 min 10min 15 min 15min 15min 15min 15min | 0.85g 0.68g 0.88g 0.37g 0.94g 0.65g 1.28g (unkown residue milky-oily mass) 2.08g (98% concentrated acid) | 32.6% 26.6% 33.7% 14.1% 36% 24.9% 49.1% 79.6% | - + + + + + + + | 118.6-119.7 °C 119.5-122.1 °C 125.3-128.2 °C 126.5-128 °C 125.7-128.8 °C 121-122 °C 119.4-123.1 °C |
| 4a | 1.56 mL acetic anhydride 1.29 mL acetic acid | 1M | water | 10 min | 1.85g | 70.9% | + | 119.2-120.9 °C |
| 4b | 1.56 mL acetic anhydride 1.29 mL acetic acid | 6M | water | 10 min | 2.37 g | 90.8% | + | 108.0-108.5 °C |
| 5 | 5 mL acetic anhydride | 6M | water | 2 hr | - | - | - | Some melted at around 90 °C. Finished melting at 118 °C. |
| 6 | 5 mL acetic anhydride | 6M | water | 20 min | 2.39 g | 91.5% | - | 120.0-120.7 °C |
| 7 | 1.56 mL acetic anhydride 3.44 mL acetic acid | 6M | water | 10 min | 1.57g | 60.2% | + | 114-116 °C |
| 8* | 5 mL acetic anhydride | 6M | water | 10 min | 1.80g | 69.0% | - | 114-116 °C |
| 9 | 5 mL acetic anhydride | 6M | Room temp distilled water | 10 min | 2.97g | 114% | + | 114-123.3 °C |
|  | 4 ml acetic anhydride | 6M | - () | 15 min | 1g | 40% | - | 132-134 °C |

Using Biosynth salicylic acid. Based on visual appearance alone, the crystals are much larger than those of Xilong Scientific, the other supplier of salicylic acid. Seeing as the large scale production would utilize the salicylic acid purchased from Biosynth, we decided to conduct a trail using it.

## Discussion

The theoretical yield of aspirin is 2.61g for excess acetic anhydride and 2g of salicylic acid. This was determined based on the following calculation: mass of aspirin = 180 g/mol (molar mass of aspirin) x 2 g salicylic acid / 138 g/mol (molar mass of salicylic acid).

Trial 2 was repeated a second time due to the positive ferric chloride test for the first sample. The second sample had a much higher yield and tested negative in the ferric chloride test.

The result of trial 5 was a yellowish putty, as though the product it contained had burnt. This might be due to the extended exposure to heat, or it might be due to the small quantity used within a large round bottomed flask. It was not possible to determine its exact mass due to its dampness, nor did it fit any of the characteristics of aspirin. For continuity purposes, it was included within the table along with its ferric chloride test and had its melting point crudely measured.

As hot water induces the hydrolysis of aspirin into acetic and salicylic acids, trial 9 was designed to observe whether using room temperature water would lead to an increase in the purity. However, as evident from its sizable mass and ferric chloride test, it prevented the salicylic acid from being washed away completely, meaning this was not a suitable step to take.

Trial 4 was designed based on the following patent: https://patents.google.com/patent/US3235583A/en. This patent describes how an 18% molar excess of acetic anhydride is sufficient to arrive at a good yield of aspirin. Based on this quantity, the following calculations were made:

nacetic anhydride = 1.18 nsalicylic acid = 1.18 x 2 g / 138 g/mol = 0.0165 mol

Vacetic anhydride = n x MM / d = 0.0165 mol x 102 g/mol / 1.08 g/mL = 1.56 mL

In order to dilute the reaction mixture, it was calculated using the proportion that for 140 g of salicylic acid, 95 g of acetic acid were used. This calculation netted that for 2 g of salicylic acid, 1.36 g of acetic acid should be used, which equates to a volume of 1.29 mL.

The ferric chloride test indicated that both trials 4a and 4b contained salicylic acid. One possible reason behind this is the combined volume of the solvents is approximately 3 mL, and the increased concentration relative to the other trials may have prevented the salicylic acid from being washed away completely. Trial 7 was therefore designed to account for this possibility, yet its result was also positive for salicylic acid. This may imply that a repetition is required. Regardless, this allows for the opportunity to conserve the usage of the reactants.

The melting point is relatively low for all the trials regardless of whether they tested positive or negative for the ferric chloride test. It was also measured after leaving the samples to dry overnight in the refrigerator. To attempt to purify 4b, two trials for a third recrystallization were conducted. 0.5g of crude aspirin were used. The first trial involved adding 2 mL of ethanol and 15 mL of water and heating until all aspirin had dissolved. The second trial involved adding 17 mL of warm water only. Both trials resulted in samples that still tested positive in the ferric chloride test, with a melting point that was not significantly different from the parent material.

| Trial | Solvent | Agitation | Mass obtained | Melting Point |
| --- | --- | --- | --- | --- |
| 4b | 2mL ethanol and 15mL water | No | 0.39 g | 108.0-108.5 |
| 4b | 17mL water | No | 0.40 g | 107.1-107.8 |

## Conclusive Remark

Overall, using an increased concentration of sulfuric acid has a positive effect on results by significantly increasing the yield. This is expected due to its catalytic activity expediting the reaction and driving it closer to completion. However, most samples had a relatively low melting point compared to the reported melting point of pure aspirin (135oC), as well as the melting point of medicinal aspirin (~130oC). Medicinal aspirin contains impurities such as maize starch, cellulose powder, methacrylic acid, and other coating reagents or those that aid in the absorption of aspirin. As such, it is expected that its melting point is depressed.

Following steps include using toluene as a recrystallization agent instead of water. This would preclude the hydrolysis of aspirin. Extra recrystallization steps may also be conducted on the samples obtained, particularly those that tested negative for the ferric chloride test.

Noting the inconsistencies across the two tests in trial 2, in order to ensure validity and reproducibility of these tests, it is worth repeating each trial three times

# Part II: Penicillin Lab Scale Production and Quantification

## Penicillin Production and Quantification May2023

Trial 2:

according to the sensitivity test :

_F1(Aqueous phase after the  organic solvent extraction)

_F2(Organic phase After PB extraction)

_F3(the final solution obtained after organic solvent extraction)

### Penicillin Quantification (May 2023)

The procedure for quantification is the same as that mentioned before using disc diffusion method leading to a calibration curve helping to calculate the concentration of our crude PenG obtained from F1 and F3 solution:

Quantification of Crude PenG obtained from F1 crystallization:

Table1 : Measurements of diameters of bacterial growth inhibition zones for different concentrations of standard dilute commercial penicillinG and F1.

| Diameter(cm) | 2.8 | 3 | 3.3 | 3.4 | 3.6 | 3.8 | 4.9 | 5.7 | 6.2 | 0.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogC | 1 | 1.2 | 1.3 | 1.39 | 1.47 | 2 | 3 | 4 | 5 | ? |
| Concentration(mg/ml) | 10 | 16 | 20 | 25 | 30 | 102 | 103 | 104 | 105 |  |

Graph 1: Graph showing the variation of penicillin concentration (LogC) as function of the diameter of the inhibition zone

According to the calibration curve y=1.15x+(-2.43)

So the concentration of PenG in F1 is 0.04mg/ml

Quantification for Crude PenG obtained from F3 crystallization

Table 2 : Measurements of diameters of bacterial growth inhibition zones for different concentrations of standard dilute commercial penicillinG and F3 solution.

| Diameter(cm) | 3.8 | 3.5 | 3.4 | 3.2 | 3.1 | 0.7 |
| --- | --- | --- | --- | --- | --- | --- |
| LogC | 1.47 | 1.39 | 1.30 | 1.20 | 1 | ? |
| Concentration(mg/ml) | 30 | 25 | 20 | 16 | 10 |  |

Graph 2 : Graph showing the variation of penicillin concentration (LogC) as function of the diameter of the inhibition zone.

According to the calibration curve y=0.623x+(-0.847)

So the concentration of PenG in F3 is 0.388mg/ml ~ 0.4 mg/ml.

Table showing every step with its duration and working date

| Steps of preparation | Working Date | Duration(Days) |
| --- | --- | --- |
| Penicillium culture | April 19 | 4 days |
| Fermentation | April24 to May3 | 10 days |
| Purification: filtration | May4 to May5 | 2 days |
| Crystallization | May5 to May12 | 7 days |
| Sensitivity test | May15 to May16 | 2 days |
| Quantification/results F1 | May15 to May16 | 2 days |
| Sensitivity test Quantification/result F3 | May18 to May 19 | 2 days |

Trial 3:

Same protocol reused in this trial (except: adding PAA : Phenylacetic acid every 24h at 30˚C for the first 48h then decrease the temprature to 25˚C for the rest of the fermentation).

PS : due to disfunction of the incubator we couldn't maintain the temprature ( between 25-30˚C) and solubility of the PAA

## Results

| Diameter (cm) | 2.6 | 2.6 | 2.7 | 2.9 | 3 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Log C | 1 | 1.2 | 1.3 | 1.39 | 1.47 | -0.3 |
| Concentration (mg/ml) | 10 | 16 | 20 | 25 | 30 | 0.5 |

Word-docx-Stage-report-ghinwayounes-2023

trial 4

Same protocol reused in this trial (except:aeorbie respiration and medium: without glucose , lactose 3g ).

### Results

| Diameter (cm) | 2.4 | 2.2 | 2.1 | 1.9 | 1.75 | 1.1 |
| --- | --- | --- | --- | --- | --- | --- |
| Log c | 1.47 | 1.39 | 1.30 | 1.20 | 1 | 0.89 |
| Concentration(mg/ml) | 30 | 25 | 20 | 16 | 10 | 7.76 |

Graph showing the variation of penicillin concentration (LogC) as function of the diameter of the inhibition zone.

# Part III: Ampicillin Lab scale Production

## Ampicillin Production and Quantification 2023

Ampicillin Synthesis Steps

Preparation of the solution of ampicillin synthesis from our crude produced penicillin:

- Measure a volume of 7.5 mL from our produced penicillin(reserved in PB solution), using a graduated cylinder

- Add 0.8g of penicillin acylase (PGA)

- Agitating for 1h

- Add 7.5ml of the ester D-(-)-a-Phenylglycine methyl ester hydrochloride, D-PGMEH (0.24g ester in 10 ml potassium phosphate buffer) to the mix

- Add 0.24g enzyme (PGA) again

- Adjust the PH for about 6.4-7 by adding NaOH

- Put the beaker containing the bar magnet on the magnetic stirrer for 22.5 hours

(Take 1ml of the filtrate for the bacterial sensibility test later A1)

Ampicillin purification and harvesting:

- Add few drops of H2SO4 to stop the enzymatic reaction and adjust the PH to 2

- Filtrate the mix using a funnel fitted with filter paper

- Add butyl acetate solvent (1Vsolvant/2Vfiltrate)(4ml/8ml)

- Let rest for 2 minutes, then the organic phase is removed and the aqueous phase is discarded after decantation

(Take 1ml from aqueous phase A2 and organic phase A3 to test the presence of ampicillin later)

- Add 2% phosphate buffer (V/V)(4ml/4ml) to the organic solution

(we obtain an aqueous phase B1 and an organic phase B2)

- Adjust the PH to 7.5 by adding NaOH

Crystallization:

- Add 2% (W/V)(~0.1g/4.3ml) NaHCO3 to the aqueous phase medium

- Cool them at 4oC for about 7 days

- Filtrate them to harvest ampicillin sodium salt

Ampicillin Quantification (Standard Protocol):

- The one bacterial strain which can be used: E. coli

- Regeneration of Escherichia coli Bacteria

Place the isolated colony of E. coli on a new standard agar petri dish and then spread it

Incubate the bacteria for approximately 18 hours in an incubator at 37°C

- Qualitative antibiogram, using disc diffusion method

To test the effectiveness of the antibiotic on the bacteria

- Take 3 to 5 colonies of the isolated colonies of E. coli with a loop

- Add them in 2ml sterile saline (NaCl 0.9%)

- Vortex the saline tube to create a smooth suspension

- Adjust the turbidity of this suspension to a 0.5 McFarland standard by adding more organism if the suspension is too light or diluting with sterile saline if the suspension is too heavy.

- Inoculate the surface of Mueller Hinton agar plate by streaking the swab 3 times over the entire agar surface

- Sit the plate at room temperature at least 3 to 5 minutes (but no more than 15 minutes) to let the surface of the agar plate dry before proceeding to the next step

- Reverse the plate and divide it into 6

- Deposit 5 disc in each quadrant with an empty quadrant used as control negative

- Add 20µl of each simple on a disc

- Reverse the plate and incubate it at at 37o for 18 to 24 h

Results and Discussion

The sensitivity test shows an important zone of inhibition in A3 and B2 proving the presence of crude produced ampicillin obtained from our crude penicillin so the reaction  has succeeded.

The test was validated while repeated twice.

Samples was taken from:

A1: solution obtained after the first filtration before adding the organic solvent

A2: Aqueous phase after butyl acetate extraction

A3: Organic phase after butyl acetate extraction

B1: Aqueous phase after PB extraction

B2:Organic phase after PB extraction

The results show :

- no zone of inhibition in A1(sample taken before adjusting the 2)

- No zone of inhibition in the aqueous phaseA2 so the total produced ampicillin is extracted into the organic phaseA3

- A small zone of inhibition in A3 so there is a loss while extracting ampicillin from this phase

- No zone of inhibition in the Aqueous phaseB1 with an important one in the organic phaseB2 so PB isn't the right solvent for ampicillin extraction for crystallization.

Crude Ampicillin quantification

Preparation for commercial ampicillin quantification

While the commercial ampicillin used in quantification had no results on E.coli

We decided to use amoxicillin instead since it has an identical antibacterial activity with ampicillin.

Crude Ampicillin quantification

Preparation for commercial ampicillin quantification

While the commercial ampicillin used in quantification had no results on E.coli

We decided to use amoxicillin instead since it has an identical antibacterial activity with ampicillin.

- Weigh 1g of commercial amoxicillin

- Add 5 ml of potassium phosphate buffer to the amoxicillin

- Filtrate the mix using a sterile funnel and filter paper

- Store the mix (C standard amoxicillin=200 mg/ml) in the fridge for later use

- Prepare the sterile tubes with different concentration 20, 15, 12, 10, 8 et 5 mg/mL

| TUBES | Solution ml | Distilled water ml |
| --- | --- | --- |
| Tube1 20mg/ml | 1 | 9 |
| Tube2 15mg/ml | 7.5 | 2.5 |
| Tube3 12mg/ml | 6 | 1.5 |
| Tube4 10mg/ml | 5 | 1 |
| Tube5 8mg/ml | 4 | 1 |
| Tube6 5mg/ml | 2.5 | 1.5 |

Quantification of the produced ampicillin using the disc diffusion method:

- Take 3 to 5 colonies of the isolated colonies with a loop, and we added them in 2ml sterile saline (NaCl 0.9%)

- Vortex the saline tube to create a smooth suspension.

- Adjust the turbidity of this suspension to a 0.5 McFarland standard(annex)

  - by adding more organism if the suspension is too light or diluting with

  - sterile saline if the suspension is too heavy.

- Use this suspension within 15 minutes of preparation.

- Inoculate the surface of 3 Mueller Hinton agar plate by streaking the swab 3 times over the entire agar surface, we rotated the plate approximately 60֯ each time to ensure an even distribution of the inoculum (use a control plate with E. coli on mueller Hinton agar)

- Leave the plates at room temperature at least 3 to 5 minutes (but no more than 15 minutes) for the surface of the agar plate to dry before proceeding to the next step.

- Reverse the plates and divide it into 4

- Deposit a disc in each quadrant

- Add 20µl of each concentration on a disc with the unknown one (A3 and B2)

- Reverse the plate and incubate it at at 37o for 18 to 24 h

- After the growth time, measure the zone of inhibition that had appeared using a ruler.

- Draw a graph showing the concentration of amoxicillin as a function of the diameter in order to be able to quantify the produced ampicillin (diameter as a function of Log C).

Results

Table 1 : Measurements of diameters of bacterial growth inhibition zones for different concentrations of standard dilute commercial amoxicillin and A3,B2.

| Diameter(cm) | 6 | 2.7 | 3.5 | 3.2 | 3.4 | 3.1 | 1(A3) | 1.1(B2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogC | 1.30 | 1.17 | 1.07 | 1 | 0.90 | 0.69 | ? | ? |
| Concentration(mg/ml) | 20 | 15 | 12 | 10 | 8 | 5 | ? | ? |

Graph 1 : Graph showing the variation of amoxicillin concentration (LogC) as function of the diameter of the inhibition zone.

According to the calibration curve y=1.22x-2.97

So for an inhibition zone of 1cm the concentration is 0.01 mg/ml

For an inhibition zone of 1.1 cm the concentration is 0.02 mg/ml

Results :

Trial 2 (6/6/2023)

Table 2: Measurements of diameters of bacterial growth inhibition zones for different concentrations of standard dilute commercial amoxicillin and A3,B2.

| Diameter(cm) | 2.1 | 2 | 1.9 | 1.75 | 1.8 | 1.55 | 1(A3) | 0.85(B2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LogC | 1.30 | 1.17 | 1.07 | 1 | 0.90 | 0.69 | ? | ? |
| Concentration(mg/ml) | 20 | 15 | 12 | 10 | 8 | 5 | ? | ? |

Graph 2 : Graph showing the variation of amoxicillin concentration (LogC) as function of the diameter of the inhibition zone.

According to the calibration curve y=1.08x-0.968

So for an inhibition zone of 1cm the concentration is 1.31 mg/ml

For an inhibition zone of 0.85 cm the concentration is 0.89 mg/ml

Note: -We use the commercial amoxicillin instead ampicillin

-We filtrated the commercial amoxicillin

-Uncompleted inhibition in B2

-We should do a double number of Petri dish in target to reduce the error.

# Part IV: Generic Pilot Plant Design (Penicillin/Ampicillin/Aspirin)

## Mechanical design

### MEGBI penicillin/ampicillin/asprin pilot scale production old design

|  | FreeCAD MEGBI pen/amp/asp pilot scale version 2015 |
| --- | --- |

### update for pilot plant ASPIRIN/PENICILLIN production 18/11/2023

### update for pilot plant ASPIRIN/PENICILLIN production 23/11/2023

|  | FreeCAD design ASPIRIN/PENICILLIN Pilot plant 23/11/2023 |
| --- | --- |

### MEGBI penicillin/ampicillin/aspirin pilot scale production FreeCAD design 28/11/2022

|  | Blue tanks : Aspirin, Green tanks : Penicillin/Ampicillin FreeCAD design MEGBI Pen/Amp/Asp pilot scale version 28/11/2022 |
| --- | --- |

### MEGBI Penicillin/ampicillin/aspirin pilot scale production FreeCAD design 16/12/2022

|  | Blue tanks : Aspirin, Green tanks : Penicillin/Ampicillin FreeCAD design Pen/Amp/Asp pilot scale version 16/12/2022 |
| --- | --- |

### MEGBI Penicillin/Ampicillin/Aspirin pilot scale production FreeCAD design 23/01/2023 (achieved)

|  | Blue tanks: Aspirin, Green tanks: Penicillin/ Ampicillin FreeCAD design Pen/Amp/Asp pilot scale version 23/01/2023 |
| --- | --- |

### MEGBI Aspirin filter and crystallizer pilot scale FreeCAD design 09/01/2023

|  | FreeCAD design aspirin filter/crystallizer pilot scale version 10/01/2023 |
| --- | --- |

### Sizes of filter/crystallizer aspirin pilot plan

# Part V: Aspirin Pilot Plant

## Requirements For Aspirin Pilot Plant Production

System requirements :

- Aspirin Pilot Plant shall be able to produce the aspirin.

- The control panel shall be able to control all the pumps, valves, mixers and read the data of the sensors (Temperature-pressure).

Physical requirements :

- The pipes shall be able to withstand the temperatures and pressures that exist at the points.

- Temperature that shall be withstood: 80 degrees celsius.

- Pressure that shall be withstood: 2 bar.

- The tanks shall be able to withstand the Temperatures exchanges, pressures and mechanical forces that exist at the points.

- Temperature that shall be withstood: 80+ degrees celsius.

- Pressure that shall be withstood: 2 bar.

-mechanical force: mixer movements and rotation.

Chemical requirements :

- The Tanks system shall be able to insulate the chemical reagents.

- The Tanks system shall be able to withstand the corrosion with H2SO4 and acetic acid.

- The pipe system used shall be able to withstand the corrosion with H2SO4 and acetic acid.

- The valves shall be able to withstand the corrosion with H2SO4 and acetic acid.

Mechanical requirements :

- The Tank system shall be made of Stainless Steel 316.

- The Tank system shall be able to close the system completely.

- The pipes shall be made of stainless steel 316.

- The pipes shall be able to resist the pressure without let gas exit through.

- The valves shall be made of stainless steel 316.

- The valves shall be able to close completely.

- The valves shall be able to open or close with independent pressure.

- The aspirin pilot plant shall be designed acccording to the mechanical design.

Safety requirements :

- The system shall be enclosed completely to ensure no toxic gas and to avoid contact with the burns from concentrated acid (H2SO4).

- The system shall be enclosed during the sterilization step to avoid the burns form hot vapor used to sterilization.

## Pilot plant system design

### Dynamic View of Aspirin Pilot Plant Production

02/18/2023 Word file contain the E-draw file of Appoximate View for aspirin pilot plant

02/18/2023 word file contain the E-draw file of Dynamic view for aspirin pilot plant

#### Operation Sequence

##### Zoomed View:

## Aspirin pilot plant Mechanical Realization

### Stand installation (18/01/2023-07/04/2023)

#### Wheel stand installation :(19/01/2023)

Autoclave place editing :(20/01/2023)

Bath for tank 2 (cooler) + tank 3 (crystallizer) and testing :(22/01/2023)-(04/07/2023)

Crystallizer and Filter equipements:(07/03/2023)

Cover for vacuum filter :(09/03/2023)

crystallizer tank and the filter tank :(09/03/2023)

Sieve and the filter con :(13/03/2023)

Vacuum filter realisation :(17/03/2023)

Crystallizer realisation :(21/03/2023-07/04/2023)

Tank 1 (The Heater) realisation:(07/04/2023)

Tank 6 (Waste Tank) realisation:(07/04/2023)

Pipes stainless steel realisation:(02/05/2023)

Sensors stainless steel instalation:(02/05/2023)

Reactor for aspirin:(20/10/2023)

#### Pilot plant mechnical realization last form: (11-13-2023)

## Test specification

### Autoclave System Test Specification

Pre-Starting

Please read these instructions thoroughly. This will make sure you obtain full safe use, Keep this instruction manual in a handy place for future reference.

Filling the tank

- Make sure all valves are closed

- Make sure the power is turned off

- Connect the water valve to autoclave

- Open the water to fill the tank (amount of water should be between 60-70%)

- Close the valve for water filling

### Safety precaution

The hot water (121 °C) could suffer of second-degree burns

### Autoclave operation sequence

1-Ensure all sanitary connections

2-Fill the autoclave tank with water

3-Plug the control system to electricity

4-check the control system if it works properly (Check if the valves works by turning them ON and OFF)

5-Operate the heater

6- Wait till the water transforms to steam (T= 121°C / P= 20000 Pa), operate the autoclave valve system to open

7-After finishing, Operate the pipe to open (to decrease the pressure)

### Autoclave System testcases

#### 001: test the resistor of the autoclave

| Step | Step Description | Expected Result |
| --- | --- | --- |
| Precondition | System is off |  |
| Switch on the system | Turn on the heater | The water degree starts to going up to reach 121°c |
| Switch off the system. | Turn off the heater | The water will remain warm and could last for 5-6h |
| Postcondition | System is off |  |

#### 002: AUTOCLAVE SYSTEME TEST OF PENICILLIN PRODUCTION

| Step | Step Description | Expected Result |
| --- | --- | --- |
| Precondition | System is OFF |  |
| open/close the valves | open all valves from the control panel and reclosed them after few minutes | Release the air from the system |
| Switch on the heater | Turn on the heater from the control panel | THE SYSTEM IS heating the water till reach 121°C |
| Sterilization of the fermenter | Open the autoclave valve (autolave--->Autoclave valve) | The steam is filling the fermenter |
| Sterilization of the whole system | Open the fermenter valve to sterilize last 2 tanks (autolave--->fermenter--->fermenter valve) | Pressure should reach 2 atm (20000 Pa) and temperature 121 °C |
| Open the manual discharge valve | Open the discharge valve manually | Pressure should decrease and the steam vent off the system |
| CLose the valves | close ALL the valves from the control panel and the discharge valve manually | ALL the valves are closed |
| switching OFF the system | switch off the system | the system is OFF |
| Postcondition | system is OFF |  |

### Aspirin Production System Test Specification

#### Pre-Starting

Please read these instructions thoroughly. This will make sure you obtain full safe use, Keep this instruction manual in a handy place for future reference.

#### Prepare the reactor

- Make sure all valves are closed

- Make sure the power is turned off

- Connect the reactif valve to the reactor

- Put the amount needed of the reactifs in the reactor (amount of reactifs: 1013.5g of salicylic acid, 2.5L acetic anhydride and 125ml concentrated H2SO4)

- Closed the valve for reactifs filling.

#### Safety precaution

-The hot water (80 °c) could suffer some burns (tank number 2)

-Concentrated Sulfuric acid causes severe skin burns and eye damage. (Wear protective gloves/protective clothing/eye protection/face protection).

### Aspirin Production operation sequence

1-Ensure all sanitary connections

2-Put the reactifs in the reactor (salicylic acid, acetic anhydride and H2SO4)

3-Plug the control system

4-check the control system if it's works properly

5-Operate the mixer to mix the reactifs in the reactor

6-Operate to add warm water to the mixture

7-Open the valves to reach the filtration system

8-Operate to open the vacuum pump in the filtration system

9-Operate the valves to refiltration the aspirin

7-After finishing, Operate the pipe to close.

### Aspirin Production System testcases

#### 003: AUTOCLAVE SYSTEME TEST OF ASPIRIN PRODUCTION

| Step | Step Description | Expected Result |
| --- | --- | --- |
| Precondition | System is off |  |
| open/close the valves | open all valves from the control panel and reclosed them after few minutes | The air is Released from the system |
| Switch ON the heater | Turn on the heater from the control panel | THE SYSTEM IS heating the water till reach 134°C |
| Sterilization of the reactor (tank 1) | Open the autoclave valve (autolave--->Autoclave valve) | The steam is filling the reactor (tank 1) P= 2 Bar (2000 mBar)/T= 134 °C |
| Sterilization of the cooler (tank 2) | Open the reactor valve V1 (tank 1) to sterilize the cooler (autolave--->Reactor--->Reactor valve V1) | The steam is filling the reactor (tank 1) and the cooler (tank 2) P= 2 Bar (2000 mBar)/T= 134 °C |
| Sterilization the tank 3 (HOT WATER TANK) | Open the tank 3 (HOT WATER TANK) valve V6 to sterilize tank 3 (HOT WATER TANK---->HOT WATER TANK valve V6) | The steam is filling the reactor (tank 1), the cooler (tank 2)and the HOT WATER TANK (tank 3) P= 2 Bar (2000 mBar)/T= 134 °C |
| Sterilization of the crystallizer (tank 4) | Open the tank 2 (cooler) valve V2 to sterilize tank 4 (crystallizer) (crystallizer---->crystallizer valve V2) | The steam is filling the reactor (tank 1), the cooler (tank 2), the HOT WATER TANK (tank 3) and the crystallizer (tank 4) P= 2 Bar (2000 mBar)/T= 134 °C |
| Sterilization of the vacuum filter (tank 5) | Open the valve V3 to sterilize tank 4 (Crystallizer) to the tank 5 (Filter) (crystallizer---->crystallizer valve V3) | The steam is filling the reactor (tank 1), the cooler (tank 2), the HOT WATER TANK (tank 3), the crystallizer (tank 4) and the vacuum filter (tank 5) P= 2 Bar (2000 mBar)/T= 134 °C |
| Sterilization of the waste tank (tank 6) | Open the valve V4 tank 5 (Filter) to sterilize tank 6 (Waste) (Filter---->Filter valve V4) | The steam is filling the reactor (tank 1), the cooler (tank 2), the HOT WATER TANK (tank 3), the crystallizer (tank 4), the vacuum filter (tank 5) and the waste tank (tank 6) P= 2 Bar (2000 mBar)/T= 134 °C |
| Open the manual discharge valve | Open the discharge valve manually | Pressure should decrease and the steam vent off the system |
| Switch OFF the heater | Turn OFF the heater from the control panel | the heater is OFF |
| CLose ALL the valves | close ALL the valves from the control panel and the discharge valve manually | ALL the valves are closed |
| switching OFF the system | switch off the system | the system is OFF |
| Postcondition | system is OFF |  |

#### 004: ASPIRIN PRIDOUCTION SYSTEME TEST

| Step | Step Description | Expected Result |
| --- | --- | --- |
| Precondition | System is OFF |  |
| TURNING ON the system | Turn ON the control panel | The system is ON |
| Switch on the mixer 1 (tank 1: Reactor) | Turn on the mixer 1 from the control panel (Reactor--->Mixer ON) | Mixing the reagents to obtain the mixture in the reactor (tank 1) |
| Open the valve V1 (tank1 : Reactor) | Open the valve V1 to transfer the mixture from tank 1 (reactor) to tank 2 (cooler) (Reactor---->Reactor valve V1) | The mixture is transferred to cooler (tank 2) |
| Open the valve 6 (tank 3 : HOT WATER) | Open the valve V6 to add water from HOT WATER TANK (tank 3) to the cooler (tank 2) (HOt WATER TANK---->HOT WATER TANK valve V6) | The warm water is in the cooler (tank 2) with the mixture |
| Open the valve V2 (tank 2: Cooler) | Turn ON the pump 1 Open the valve V2 to transfer the Water from cooler (tank 2) to crystallizer (tank 4) (cooler---->cooler pump 1) (cooler---->cooler valve V2) | The reagents are transferred to the crystallizer (tank 4) and formation of crystals |
| Switch on the mixer 2 (tank 4 : Crystallizer) | Turn on the mixer from the control panel (Crystallizer--->Mixer ON) | Mixer 2 is ON and start Mixing the reagents in the Crystallizer (tank 4) |
| Open the valve V3 (tank 4: Crystallizer) | Open the valve V3 to transfer the Water from crystallizer (tank 4) to the filter (tank 5) (crystallizer---->crystallizer valve V3) | The mixture transferred to the filter (tank 5) |
| Switch on the vacuum pump | Turn on the vacuum pump from the control panel (Filter---->Filter pump ON) | The air is vacuumed in tank 6 (Waste tank) |
| Open the valve V4 (tank 5 : Filter) | Open the valve V4: 1-the vacuum filtration begin 2-Aspirin crystals are now in the filter (tank 5) (Filter---->Filter valve V5) | The filtrate transferred to the waste tank (tank 6) |
| Open the valve V7 | Open the valve V7 to add 30L warm water from WARM WATER TANK (tank 3) to vacuum filter (tank 5) (WARM WATER TANK---->WARM WATER Valve V7) | Crystals re-dissolved with 30L warm water after 20min in the vacuum filter (tank 5) |
| Open the valve V5 and pump 2 (tank 5 : Filter) | Open the valve V5 The solution is pumped from tank 5 to tank 4 for the second crystallization (vacuum filter---->vacuum filter Valve V5) | Solution transferred to the crystallizer (tank 4) and start the crystallization |
| Switch on the vacuum pump for the second filtration | Turn on the vacuum pump from the control panel (Filter---->vacuum pump ON) | The air is partial vacuumed in tank 6 |
| Open the valve V3 and V4 | Open the valve V3 and V4: 1-the vacuum filtration begin 2-Aspirin crystals are now in the vacuum filter (tank 5) (1-crystallizer---->crystallizer valve V3) (2-vacuum filter---->vacuum filter valve V4) | The filtrate transferred to the waste tank (tank 6) |
| switching OFF the system | switch OFF the system | the system is OFF |
| Postcondition | system is OFF |  |

## System Tests

In 2 separate docx file for download:

MEGBI Annual Report 2023 (Asprin Pilot Plant System Tests 1 (including videos) (docx))

MEGBI Annual Report 2023 (Asprin Pilot Plant System Tests 2 (including videos) (docx))

## Cleaning/Tableting/Recycling Pilot plant

### Cleaning process

Tank cleaning is the most essential in the industries. There is usually a buildup of contamination on the reactor walls and on the agitator blade surface, which can be easily cleaned with different methods.

- Hydro -blasting:

Hydro- blasting can be utilized to clean the insides of pipes, tanks, and process reactors.

Our process uses high pressure water jetting (15,000 psi to 50,000 psi) to remove scaling from the internal surfaces of pipe. This process is performed on steam piping prior to steam and air blowing of the steam lines to reduce the time, water, and fuel required to clean the steam piping.

The nozzle can be rotated 360° on a horizontal or vertical plane to form a crisscross pattern to thoroughly clean the tank and remove the stickiest residue

- cryogenic cleaning

Dry ice cleaning consists of projecting particles of ice or dry ice, solid CO2 at -78°C, by a flow of compressed air onto a surface to be cleaned. The combination of intense cold and mechanical shock causes the dirt to detach from its support.

- the particles of ice or dry ice are accelerated by a flow of compressed air.

- localized thermal shocks weaken the pollutants

- cracks form and the pollutant cracks with the cold

- the micro- excavators strike the surface at a speed of 300 m/s and remove the loosened pollutants

- chemical cleaning

Cleaning is achieved by physical action of high velocity flow, jet sprays, agitation and chemical action of cleaning agents enhanced by heat. While mechanical forces are necessary to remove gross soil and to ensure adequate penetration of cleaning solutions to all areas.

three steps: initial rinse, cleaning with detergent(s) and final rinse.

- Detergent: Alcon ox™ Liquinox™

Use to clean: pharmaceutical apparatus, industrial parts, pipes,tanks and reactor.

Surfaces cleaned: Corrosion inhibited formulation recommended for

glass, metal, stainless steel.

Directions: Make a fresh 1% solution (2 1/2 tbsp per gal, 1 1/4 oz per

gal, or 10 ml per L) in cold, warm or hot water. If available, use warm

water. Use cold water for blood stains. For diffi cult soils, raise water

temperature and use more detergent. Clean by soak, circulate,

wipe…RINSE THOROUGHLY—preferably with running water.

- CIP process:

- STEP 1: PRE-RINSE

The pre-rinse is a very important step in the CIP

The pre-rinse cycle:

- Wets the interior surface of the lines and tank

- Removes most of the remaining residue

- Dissolves sugars and partially melts fats

- Provides a non-chemical pressure test of the CIP flow path

Use potable plant water, de-ionized water (DI).

A Turbidity Sensor may be used to verify that the pre-rinse effectively

removes all solids.

- STEP 2: CAUSTIC WASH – (140° – 185° F)

Caustic washes soften fats, making them easier to remove. Also known as caustic soda, sodium hydroxide or NaOH.

Caustic is typically used as the main detergent in most CIP wash cycles. A non-foaming formulation can help reduce pump cavitation and increase efficiency.

Water Saving Tip: In many cases, the caustic wash can be returned to its tank and re- used multiple times, which significantly reduces water, chemical, and energy costs over a single tank system.

- STEP 3: INTERMEDIATE RINSE

Fresh water flushes out residual traces of detergent remaining from the caustic wash.

- Level Transmitters and Probes monitor tank levels of wash and rinse tanks.

- Flow Transmitters ensure optimum flow for spray devices to precisely control wash and rinse steps.

- Conductivity Transmitters ensure chemical levels are hitting predetermined set point.

- STEP 4: FINAL RINSE

Rinse with either DI, RO, or city water to flush residual cleaning agents.

Note! the final rinse water may be recovered and reused as the pre-rinse solution for the next cleaning cycle.

- STEP 5. SANITIZING RINSE

May be required to help kill microorganisms before starting the next production run. The PAA solution:

- Is a strong disinfectant even at low temperatures.

- Rinses away while leaving little or no chlorine residue to corrode stainless steel.

- Is effective against all microorganisms including spoilage organisms, pathogens and bacterial spores.

- It has also proven to be eco-friendlier in the wastewater stream.

- Has a strong, pungent odor so it should only be used in well-ventilated areas.

- Warning: Sanitizers reduce bacterial growth but don’t completely kill all pathogens in the system; since it is the last step in the cleaning process, re- circulating the sanitizing solution could run the risk of spreading any leftover contamination that might be present. Sanitizers can also be sensitive to high temperatures and can lose their effectiveness fairly rapidly once they are in solution.

Note! Peracetic acid should be safe with stainless steel

finally we decide to use hydro-blasting method by dividing this method into three steps : pre-rinse addition of detergent, and final rinse.

### Aspirin tableting process

Tableting aspirin is beneficial because it makes the medication easier to swallow and ensures a consistent dose. It also helps to prevent the medication from breaking down too quickly, allowing it to be stored for longer periods of time.

-First method:

Procedure:

A stable aspirin tablet may be prepared under the conditions where RH ( Relative humidity is amount of water present in an air particle over can be measured by hygrometer) is maintained below 30 %, by the process comprising steps of:

- Sifting aspirin, and microcrystalline cellulose or corn starch.

- Blending the material of step 1.

- Sifting the pregelatinized (Examples of binders include pregelatinized starch: polyvinyl pyrrolidone) and Silica colloidal anhydrous (Example of lubricant and glidant) and adding to material of step 2.

- Sifting the stearic acid and adding to material of step 3 and blending.

- Compressing the blend of step 4, The above blend was compressed using approved punches and dies.

- Preparing the enteric coating dispersion by adding and mixing talc hydrated magnesium silicate (apply a safe barrier against contamination as a glidant to improve powder flow in tablet compression), meth acrylic acid-ethyl acrylate copolymer (1:1), triethylcitrate and simethicone emulsion in water

- Spraying the dispersion onto the tablet.

Formula:

| Ingredients | Amount (mg) | Role |
| --- | --- | --- |
| Aspirin | 75.00 | Active ingredient |
| Corn starch | 24.20 | Disintegrant |
| Starch Pregelatinized | 7.70 | Binder |
| Silica Colloidal Anhydrous | 2.20 | Anticaking agent, emulsion stabilizer, glidant |
| Stearic Acid | 0.90 | Aspirin against degradation |
| Talc | 3.33 | Giladant |
| Meth acrylic Acid-Ethyl Acrylate Copolymer (1:1) | 10.50 (As dry substance) | Tablet binder, tablet coating agent |
| Triethyl Citrate | 1.05 | It allows a slower release of the contents of the tablets |
| Simethicone Emulsion | 0.12 (As dry substance) | Emulsion |
| Purified Water | q.s (Quantum satis=the amount which is enough) | used in Preparation of enteric coating |

-Second method

Requirement:

- Chemicals: Aspirin, starch, polyvinyl pyrrolidone, ethanol, magnesium stearate, talc.

- Glass wares: measuring cylinder, beaker, mortar and pestle, granulating sieve…

- Equipment: tray dryer, tablet press.

Procedure:

1.  Weight and pass aspirin and starch powder through 60#sieve (size with a 0.0098" (250μm) nominal sieve opening with a typical wire diameter of 0.16mm).

2.  Mix aspirin and starch uniformly in mortar and pestle

3.  Prepare 10% PVP solution in ethanol and stir until it becomes clear

4.  Add PVP solution dropwise in mortar to get cohesive mass

5.  Screen prepared cohesive mass through 10#granulating sieve (a medium size U.S. Standard mesh size with a 0.0787" (2mm) nominal sieve opening with a typical wire diameter of 0.9mm) and collect it on granulating tray

6.  Dry granules in tray dryer at 50Co for 30min

7.  Blend granules with remaining ingredients (talc and magnesium stearate) using polybag

8.  Store prepared granules in well closed and labelled container

Note: Corn starch is most suited as a vehicle for tablet compression in the pharmaceutical industry.PVP improving the bioavailability and stability of drugs, improving the physic mechanical properties of preparations, adjusting the release rate of drugs.

Formula:

Note: this table contains a mistac in the quantity taken  of aspirin (50 tablets) 15g instead of 1.5 g

To watch a video explaining click  on this link: https://www.youtube.com/watch?v=r_3bSQd2xv0&t=1453s

### Evaluation test

A.Evaluation of granules

The Prepared Tablet is Evaluated in terms of bulk density, tapped density, the angle of repose, Carr’s Index and, hardness test, weight variation test, friability test and in vitro study. The result associated with optimized batch is good satisfactory and having better drug release kinetic.

We will have used the method and the into evaluated granules: bulk density, tapped density, the angle of repose and Hausner’s ratio.

A-A volume of powder is filled into a graduated glass cylinder and repeatedly tapped for a known duration. the volume of powder after tapping is measure.

1.Bulk density=weight /bulk volume (Bulk density: This includes the volume of solid fraction of particles and intra- and interparticulate volumes, Bulk density is defined as the mass of the many particles of the material divided by the total volume they occupy).

2.tapped density =weight / tapped volume (The tapped density is an increased bulk density attained after mechanically tapping a container containing the powder sample)

The tap density tester is ideal for measuring the tapped density of powders:

B.The Hausner’s ratio is a number that is correlated to the flow ability of a powder or granular material. The Hausner ratio is used in a wide variety of industries as an indication of the flow ability of a powder.

Hausner’s ratio=Tapped density /bulk density

A Hausner Index greater than 1.2 is considered an indication of poor flow but good compressibility and good cohesion.

C. Angle of repose is a parameter commonly used for the evaluation of interparticle force, Angle of rest is defined as the angle that an inclined plane makes with the horizontal when a body placed on it just starts sliding.

How can be calculate angle of repose?

The simplest method for the determination of the angle of repose is the “poured” angle. A funnel with a wide outlet is affixed at a distance of 10 cm above a piece of paper is placed directly beneath the funnel. Powder is added while the funnel is closed. The contents flow through and collect on the paper. The diameter of the cone (D) and two opposite sides (l1 + l2) are measured with rulers. The angle of repose (θ) is calculated from the equation arc cos [D/(l1 + l2)]. The relationship between   flow properties and angle of repose has been established. !!When the angle of repose is less than 25 degrees, the flow is said to be excellent; on the other hand, if the angle of repose is more than 40 degrees, the flow is considered to be poor.

B. Evaluation of tablets

Disintegration test for tablets:

The disintegration test is used to show how quickly the tablet breaks down into smaller particles, allowing for a greater surface area and availability of the drug when taken by a patient.

To carry out a disintegration test for tablets, we use a basket which holds 1 to 6 tablets. This is then raised and lowered into a beaker of water, which is used to simulate conditions in the stomach at 37°C. If the tablets or capsules float, perforated plastic disks are placed on the top of the tablets to keep them under the water level. The tablet disintegration time is taken when no residue is left in the mesh.

### Recycling

The production of aspirin typically generates waste byproducts which can be harmful to the environment if not disposed of properly.Recycling of aspirin production waste can be done through various methods such as:

I .Recovery of salicylic acid: Salicylic acid can be recovered from the waste generated during aspirin production through a process called acidification and crystallization. Here are the basic steps involved in the process:

1.Acidification: The waste solution containing salicylic acid is first acidified with a strong mineral acid such as hydrochloric acid or sulfuric acid to convert the salicylic acid to its protonated form. The acidification is done by slowly adding the mineral acid to the waste solution while stirring until the pH of the solution reaches around 2. The protonation of salicylic acid is described by the following equation:

C7H6O3 + H3O+ → C7H7O3+ + H2O.where C7H6O3 represents salicylic acid and H3O+ represents the hydronium ion.

For example, if there is 1 gram of excess salicylic acid in the crude aspirin mixture, then approximately 1.5 to 2 grams of sulfuric acid would be added to react with the excess salicylic acid.

It's important to note that the exact quantities used can vary depending on the specific process and the conditions under which the acidification is carried out. Additionally, the acidification step must be carefully controlled to avoid the formation of undesirable byproducts or the degradation of the salicylic acid.

2.Extraction: The protonated salicylic acid is then extracted from the waste solution using an organic solvent such as ethyl acetate, dichloromethane, or toluene. The organic solvent is added to the acidified solution and the mixture is shaken or stirred for a few minutes to allow for the extraction of the protonated salicylic acid into the organic layer. The organic layer is then separated from the aqueous layer using a separating funnel. The quantities used in the extraction process (such as the amount of organic solvent or washing solution used) can vary depending on the specific process and the amount of crude aspirin being processed.

3.Recrystallization: The organic layer containing the protonated salicylic acid is then evaporated to dryness to obtain the crude salicylic acid. The crude salicylic acid is then dissolved in a suitable solvent such as ethanol or methanol and heated to dissolve the salicylic acid completely. The solution is then allowed to cool slowly to room temperature, causing the salicylic acid to recrystallize from the solution. The crystals are then filtered, washed with cold solvent to remove any impurities, and dried to obtain pure salicylic acid. the general amount used is about 3-5 times the mass of the salicylic acid being recrystallized. For example, if you have 1 gram of salicylic acid, you would use approximately 3-5 mL of ethanol as the solvent for the recrystallization step.

Once the salicylic acid has been recovered and purified, it can be reused in the production of aspirin or other products that require salicylic acid as a starting material.

II. Recycling of acetic acid:

- Reuse of acetic acid: The acetic acid produced during aspirin production can be purified and reused in subsequent batches of aspirin production. This reduces the amount of waste generated and saves on production costs,

Acetic acid is commonly used in the synthesis of aspirin as a catalyst to help facilitate the reaction between salicylic acid and acetic anhydride.

Extraction of acetic acid: in addition, acetic acid can be recovered by distillation or by extraction with an appropriate solvent such as ether or dichloromethane Once recovered and purified, acetic acid can be reused in other chemical reactions, thus reducing costs and minimizing waste from aspirin production.

Add the solvent: A small amount of solvent is added to the mixture of water     and acetic acid. Mix thoroughly: The mixture is mixed thoroughly to ensure that the solvent and water/acetic acid mixture are fully in contact with each other. Allow to separate: The mixture is then allowed to separate into two layers - the aqueous layer (containing water and acetic acid) and the organic layer (containing the solvent).

Collect the organic layer: The organic layer is carefully separated from the aqueous layer and collected in a separate container.

Repeat the extraction: The extraction process may be repeated multiple times to increase the yield of acetic acid.

Recover the acetic acid: The solvent is then evaporated from the organic layer to recover the acetic acid.

- Neutralization: The waste acetic acid can be neutralized using an appropriate base such as sodium hydroxide or calcium hydroxide. This results in the formation of a salt that can be disposed of safely. the neutralization reaction of acetic acid, which is a weak acid, with a strong base, such as sodium hydroxide (NaOH), can be represented by the following chemical equation:

CH3COOH + NaOH → CH3COONa + H2O

To determine the exact amounts needed for a given neutralization reaction, one must know the concentrations of the reactants and use stoichiometry calculations to determine the amounts needed. For example: if one wishes to neutralize 10 mL of a 0.1 M acetic acid solution with 0.2 M sodium hydroxide, one would need to add 20 mL of the NaOH solution to the acetic acid while gently shaking until neutralization is complete.

III. Recycling of acetic anhydride

In the case of extracting acetic anhydride from the waste products of aspirin production, a suitable solvent is needed that will selectively dissolve the acetic anhydride. One possible solvent for this purpose is dichloromethane (also known as methylene chloride). Here are the steps for extracting acetic anhydride from the waste:

-Add the waste mixture to a separator funnel.

-Add enough dichloromethane to the separator funnel so that the mixture is fully covered.

-Close the funnel and shake it gently to allow the solvent to mix with the waste mixture.

-Wait for the mixture to settle, with the denser layer at the bottom and the lighter layer (containing the acetic anhydride) on top.

-Slowly drain off the lighter layer (the dichloromethane layer containing the acetic anhydride) into a clean container.

-Discard the heavier layer (the aqueous layer containing the waste products).

The extracted acetic anhydride can then be further purified using methods such as distillation or recrystallization.

Recrystallization of acetic anhydride can be done by following the steps:

-Dissolve acetic anhydride in an appropriate solvent such as ethanol or acetone. The amount of solvent used will depend on the amount of acetic anhydride to be recrystallized.

-Heat the solution gently until all the acetic anhydride is dissolved.

-Slowly add the recrystallization solvent, usually water or acetone, to the hot solution, stirring constantly, until the solution is saturated and crystals begin to form.

-Allow the solution to cool slowly, preferably to room temperature, to allow crystals to form.

-Filter the crystals formed using a Büchner funnel and wash with the recrystallization solvent.

-Dry the crystals in the open air or in a low temperature oven

It is important to note that the recrystallization can be carried out several times to improve the purity of the crystals.

IV. Recycling of water: Water is an important component in the production of aspirin, and it is often used in large quantities. The water used can be treated and recycled for use in subsequent batches of aspirin production. evaporation and condensation: This process involves evaporating the waste water to remove impurities and then condensing the water vapor to produce purified water. The purified water can then be reused in the manufacturing process.

note :

Extraction of salicylic acid: It is generally extracted first because it is the main compound in the reaction mixture of the synthesis of aspirin.

Extraction of acetic acid: If you need to recover the acetic acid, you can do it after extracting the salicylic acid because it is present in lesser quantity in the reaction mixture.

Extraction of acetic anhydride: If you want to recover acetic anhydride from waste from aspirin production, this can be done after recovering acetic acid, since acetic anhydride is synthesized from acetic acid. However, please note that recovering acetic anhydride from waste can be more complicated than the first two extractions.

# Annex: The most important solutions in biology lab

## Making bacterial stock solution:27-10-2022

## Realisation:28-10-2022

The strain will be fresh to be inoculated and then frozen so E.coli and S.aureus must be renewed 24 hour before and then inoculate d in skim milk before freezing at -20oc

## Solution preparation

NaOH(1M):

- Fill the flask half way with distilled water

- Add 40g of NaOH powder

-Add water to reach the 1L mark

-finally mix it to obtain NaOH solution (1M).

H₂SO₄(1M):

-Fill the flask half way with distilled water

Add 56 ml of H₂SO₄

-add water to reach the 1L mark

-Finally mix it to obtain H₂SO₄ solution (1M).

NaCl 0.9%:

-We put 0.9g NaCl in 100 ml distilled water

-Put it in Autoclave for microbial usage.

Bleach 1%:

-We took 1ml of bleach solution

-Put it in 100ml water graduated cylinder

-Mix it to obtain Bleach solution 1%.

Muller Hinton agar:

-Measure 5.7g of Muller Hinton powder

-Put it in 150 ml of distilled water

-Mix them on magnetic stirrer with heating (to reach boiling point)

-Put it in Autoclave (1h) for microbial usage.

Standard agar:

-Measure 5.55g of standard agar powder

-Put it in 150 ml of distilled water

-Mix them on magnetic stirrer with heating (to reach boiling point)

-Put it in Autoclave (1h) for microbial usage.

Broth:

-Measure 5g peptone, 1g tryptone, 1g glucose, 1g yeast extract and 2.5g sodium chloride

-Put it in 500ml of distilled water

-Mix them on magnetic stirrer with heating (to reach boiling point)

-Put it in Autoclave (1h) for microbial usage.

Phosphate buffer (0.1M, pH:7):

-Prepare 800 ml of distilled water in a suitable beaker

-Add 9.34g of Potassium phosphate dibasic (0.05364M, mw: 174.18g/mol) to the solution

-Add 6.31g of Potassium phosphate monobasic (0.0436M, mw: 136.09 g/mol) to the solution

-Add water until the volume reach 1L

-Put it in Autoclave (1h) for microbial usage.

## Preparation of the turbidity calibration 0.5 McFarland:(40)

- we added 0.5 mL of a 0.048 mol/L solution of BaCl2 (1.175% w/v BaCl2 2H2O) to 99.5 mL of a 0.18 mol/L solution (0.36 N) of H2SO4 (1% v/v) and we shook vigorously

- We checked the density of the suspension using a spectrophotometer with a 1 cm beam and matching cuvettes. The absorbance at 625 nm should be between 0.08 and 0.13

- We distributed the suspension in tubes of the same size as those used to adjust the inoculum and then we sealed the tubes

- Once sealed, we stored these tubes at room temperature and protected from light. Before use, we mixed the tube vigorously using a Vortex (6 months’ storage)

## Prices of laboratory products:

|  | Quantity | Company supplier | Price |
| --- | --- | --- | --- |
| Tryptone | 250g | Merc | 86.3$ |
| Glucose | 1Kg | Merc | 56.5$ |
| Lactose | 500g | Ali express | 72.5$ |
| Immobilized penicillin G acylase | 10g | Biosynth | 69.3$ |

## MEGBI-Suppliers contact:

| Contact List |  |  |
| --- | --- | --- |
| Chemical reagent | Laboratory equipment | Drugs (Ministry of Health website) |
| -Biosynth.com (Shipping from Germany) | -Medi lab c.rached@medilab.com | https://www.moph.gov.lb/ar/Drugs/index/0/7963 |
| -ibra@ibrahadad.com patriciazaczac@ibrahadad.com 01-901324 01-901325 Patricia: 03-971430 | -Burhan Kabbara 03-339523 |  |
| -Vtc lab Mark@vtc-lb.com | -Firas Zakariya (CityMed) 81-879064 |  |

References

1.	Aspirin and the Salicylates - K. D. Rainsford - Google Books [Internet]. [cited 2022 Dec 7]. Available from: https://books.google.com.lb/books?hl=en&lr=&id=WQklBQAAQBAJ&oi=fnd&pg=PP1&dq=.+The+Bayer+Company+replaced+the+phenol+group+with+an+ester+group.+This+esterified+compound+(acetylsalicylic+acid,+also+known+as+aspirin)+was+shown+to+be+much+less+irritating+than+salicylic+acid&ots=5PPj0cVogz&sig=FcedfowbA_QNgoZ7q6dEIFTlHSs&redir_esc=y#v=onepage&q&f=false

2.	cycles T text provides general information S assumes no liability for the information given being complete or correct D to varying update, Text SCDM up to DDTR in the. Topic: Bayer AG [Internet]. Statista. [cited 2022 Dec 8]. Available from: https://www.statista.com/topics/4292/bayer-ag/

3.	The History of Aspirin | The International Aspirin Foundation [Internet]. Aspirin Foundation. [cited 2022 Dec 8]. Available from: https://www.aspirin-foundation.com/history/

4.	Muthuselvi C, Dhavachitra M, Pandiarajan S. Growth and Characterization of Aspirin Crystal in the Phosphoric acid Medium. Journal of Chemical and Pharmaceutical Research. 2017 Jan 1;2016:804–14.

5.	Fossum C. 8-Synthesis of Aspirin. :5.

6.	161lab_Aspirin-purityUpdatedPSGF8-23-2016-.pdf [Internet]. [cited 2022 Dec 26]. Available from: https://www.bellevuecollege.edu/wp-content/uploads/sites/140/2014/06/161lab_Aspirin-purityUpdatedPSGF8-23-2016-.pdf

7.	6_2021_09_19!09_01_35_PM.pdf [Internet]. [cited 2022 Dec 28]. Available from: https://uomustansiriyah.edu.iq/media/lectures/6/6_2021_09_19!09_01_35_PM.pdf

8.	Synthesis of Aspirin Lab [Internet]. 2011 [cited 2022 Dec 7]. Available from: https://www.youtube.com/watch?v=Y4NMpO1xI8U

9.	Synthesis of aspirin [Internet]. 2020 [cited 2022 Dec 8]. Available from: https://www.youtube.com/watch?v=fww5w381FRQ

10.	How aspirin is made - production process, manufacture, history, used, parts, procedure, steps, product [Internet]. [cited 2022 Nov 22]. Available from: http://www.madehow.com/Volume-1/Aspirin.html

11.	F. Lombard. La louse de vitesse entre les antibiotiques et le bactéries pahtogènes résistantes : analyse et discussion de qulques pistes. 2005;

12.	Jana HAMZEH. Production et quantification de la penicillin [Internet] [Master thesis]. Lebanese University, Faculty of Sciences; 2022. Available from: http://aecenar.com/index.php/downloads/send/6-megbi-institute/1172-penicillin-production-master-thesis

13.	Boukhedenna Naoual and Merouane Ilham. Production de la pénicilline V et G in vitro par Penicillium chrysogenum. 2014;

14.	Böhm J, Hoff B, O’Gorman CM, Wolfers S, Klix V, Binger D, et al. Sexual reproduction and mating-type–mediated strain development in the penicillin-producing fungus. Proc Natl Acad Sci USA. 2013 Jan 22;110(4):1476–81.

15.	Samson RA, Hadlok R, Stolk AC. A taxonomic study of the Penicillium chrysogenum series. Antonie Van Leeuwenhoek. 1977;43(2):169–75.

16.	Andersen, JC. Frisvad, Sondergaard, Rasmussen, LS. Larsen. Associations entre les espèces fongiques et les matériaux de construction endommagés par l’eau. 2011;

17.	Raper, Kenneth, Thom, Charles. A manual of the penicillia. 1949.

18.	Hoog S, Guarro J, Gené J, Figueras M. The Atlas of Clinical Fungi. Journal of Organic Chemistry - J ORG CHEM. 2005 Jan 1;2000.

19.	García-Estrada C, Martín JF, Cueto L, Barreiro C. Omics Approaches Applied to Penicillium chrysogenum and Penicillin Production: Revealing the Secrets of Improved Productivity. Genes (Basel). 2020 Jun 26;11(6):712.

20.	Canzani D, Aldeek F. Penicillin G’s function, metabolites, allergy, and resistance. nutrition-human-health [Internet]. 2017 [cited 2022 Sep 19];01(01). Available from: http://www.alliedacademies.org/articles/penicillin-gs-function-metabolites-allergy-and-resistance-7764.html

21.	Brian-Jaisson F. Identification and characterization of exopolymers from biofilms of marine bacteria. 2014 Feb 6;

22.	Michael L. Shuler, Fikret Kargi. Bioprocess Engineering Basic Concepts.

23.	Yasso Mohamed. Industrial Fed-batch Production of Penicillin G from Penicillium Chrysogenum Mold. 2018 [cited 2022 Oct 11]; Available from: http://rgdoi.net/10.13140/RG.2.2.16148.55682

24.	Torche S., Bensegueni L. Pharmacologie spéciale : Les antibiotiques. 2019;

25.	Justine Charlet. Pénicilline : comprimés, antibiotique, quelles maladies ? 2021;

26.	(PDF) Industrial Fed-batch Production of Penicillin G from Penicillium Chrysogenum Mold [Internet]. [cited 2022 Oct 19]. Available from: https://www.researchgate.net/publication/329885686_Industrial_Fed-batch_Production_of_Penicillin_G_from_Penicillium_Chrysogenum_Mold

27.	Anderson EA. nventors. Gino J. Pierotti Raymond A. Wilson. :6.

28.	McGlade E, Lennon R. BE401 Industrial Processing. :11.

29.	MEGBI-APP (Antibiotics Production Pilot Plant) Final Report (Period 2016 - 2020). 2016.

30.	Alaa BZAL. Production and Quantification of Ampicillin [Internet]. Lebanese University, Faculty of public health; 2022. Available from: http://aecenar.com/index.php/downloads/send/6-megbi-institute/1171-production-et-quantification-de-l-ampicilline-memoire-m2

31.	Recherche et dénombrement d’ Escherichia coli thermotolérants dans les échantillons solides ou semi-solides :.pdf [Internet]. [cited 2022 Feb 18]. Available from: https://www.ceaeq.gouv.qc.ca/methodes/pdf/MA705EcBCIG1.pdf

32.	Growth Requirements of E. coli and Auxotrophs - Video & Lesson Transcript [Internet]. Study.com. [cited 2022 Feb 18]. Available from: https://study.com/academy/lesson/growth-requirements-of-e-coli-and-auxotrophs.html

33.	Lynn Deaguero A. IMPROVING THE ENZYMATIC SYNTHESIS OF SEMI-SYNTHETIC BETA-LACTAM ANTIBIOTICS VIA REACTION ENGINEERING AND DATA-DRIVEN PROTEIN ENGINEERING [Internet]. Georgia Institute of Technology; 2011. Available from: https://smartech.gatech.edu/bitstream/handle/1853/42727/deaguero_andria_l_201112_phd.pdf

34.	antibiotics_fermentation_products_small_molecules_apis.pdf [Internet]. [cited 2022 Oct 13]. Available from: https://www.diaion.com/en/application/pharmaceutical/pdf/antibiotics_fermentation_products_small_molecules_apis.pdf

35.	Zhifa Y, Shuqiu YU, Jiayong C, Shouxin LIU, Chongwei Z. A NEW PROCESS FOR THE EXTRACTION OF PENICILLIN G FROM THE FILTRATE OF FERMENTATION BROTH WITH TBP IN BUTYL ACETATE. Chinese Journal of Chemical Engineering. 1992 Jun 28;7(1):83.

36.	Raahave D. Paper Disk-Agar Diffusion Assay of Penicillin in the Presence of Streptomycin. Antimicrob Agents Chemother. 1974 Nov;6(5):603–5.

37.	L. Shuler M, Kargi F. Bioprocess Engineering Basic Concepts Second Edition [Internet]. Second edition. United States: Prentice Hall PTR Upper Saddle River, NJ 07458 www.phptr.com; 2002. 576 p. Available from: https://www.academia.edu/44843690/Bioprocess_Engineering_Basic_Concepts_Second_Edition?pop_sutd=false

38.	Nandi A, Pan S, Potumarthi R, Danquah MK, Sarethy IP. A Proposal for Six Sigma Integration for Large-Scale Production of Penicillin G and Subsequent Conversion to 6-APA. Journal of Analytical Methods in Chemistry. 2014;2014:1–10.

39.	Fatima Antar, Mariam Mourad, Asia Mourad, Samer Youssef, Samar Youssef, Samir Mourad. MEGBI Antibiotics Production Pilot Plant (MEGBI-APP) - 6 th Project Report (Apr 2018 - Feb 2019) - [Internet]. 2018. Available from: http://aecenar.com/index.php/downloads/send/6-megbi-institute/463-megbi-app-report-6-2018-pdf

40.	Antibiogramme | Protocole | Interprétation [Internet]. [cited 2021 Dec 2]. Available from: https://microbiologie-clinique.com/antibiogramme.html