بسم الله الرحمن الرحيم

ELECTROLYZER

PROCESS CONTROL

SYSTEM

Produced by: Nour Karim

Raja Murad

Last update: Monday, September 20, 2021

Table of Contents

1.	Requirements and Overview	3

2.	Plant-End	6

2.1.	Automatic valves	7

2.1.1.	Water Automatic valve	7

2.2.	Gathering	7

2.2.1.	Automatic valves	8

3.	Power Control Box	9

3.1.	Wiring Diagram	10

3.2.	Level Detector	11

3.2.1.	Schematics and Principle of Operation	12

3.3.	Flow Chart and Source Code	14

4.	User-End	17

4.1.	GUI	17

4.2.	Controller	19

# Requirements and Overview

The electrolyzer process control system to be implemented should have the following technical requirements:

- The user must be able to switch between manual and auto modes.

- The user must be able to control the electrolyzer plant from a minimum distance of 50 meters.

- The user must be able to monitor different sensor data on a well-organized GUI.

- The user must be able to select the valves’ opening angles from the GUI.

- The system must have a 2-way communication channel.

In other words, the system functionality can be summarized by the following graphical representation:

The system can be divided into 3 subcategories; The plant side where the electrolyzer setup is set, the power control unit that handles all the control procedure, and the user side where all the controls are set.

Figure 1. User-end

Figure 2. power control box

Figure 3. plant-end

# Plant-End

## Automatic valves

An Automatic valve is an electromechanical device that uses an electric current to generate a magnetic field and thus actuate a solenoid that controls the opening of the fluid flow in a valve.

### Water Automatic valve

The following table presents the characteristics needed of water Automatic valve:

| Name | Type | Port Number | Control type |
| --- | --- | --- | --- |
| Electrolysis cell Water Discharge | Direct-operated Automatic valve | 3 (3-way) | Supply Voltage |
| Gas condenser Water Discharge | Servo motor | 2 (2-way) | Supply Voltage + PWM |

Figure 4. 3-way Automatic valve			Figure 5. 2-way Automatic valve

## Gathering

In this part, all missed equipment was put in its place in the whole system.

### Automatic valves

#### Gas Condenser Water Discharge

The Automatic valve that has to discharge water from condenser (H) will take place in the red rectangle, as shown in figure 4. The same thing for condenser (O).

Figure 7. Valves with/without Joints

Figure 6. M/M Joint

In order to connect Automatic valve to system, two M/M joint of hole size: 15 mm are needed, one for solenoid inlet, and one for its outlet. Figure 2 and 3, below, presents Automatic valve, and M/M joint.

Figure 7, and 8 show how these two Automatic valves was gathered with the whole system.

Level Detector

Relay Box

Power Control Box

Controller

Cell Valves Switch

Cell Power Switch

Valves, Pumps Breaker

Cell Power Breaker

Water Valve Switch (DC Valve)

KoH Pump Switch

Water Pump Switch

- Cell Power Breaker: Turns on/off the electrolyzer cell power source (12V @ 145A) and is rated at 10A.

- Valves and Pumps Breaker: Turns on/off all the system’s electrical actuators. Rated at 5A.

- Switch 1 (Cell Power): Manual at upper position in which the cell power source will turn ON directly if the breaker is ON and the switch is up. Otherwise, it will turn ON/OFF according to controller command if switched to lower position.

- Switch 2 (Cell Valves): Manual at upper position in which both cell valves will turn ON directly if the “valves and pumps” breaker is ON and the switch is in its upper position. Otherwise, it will turn ON/OFF according to controller command if switched to lower position.

- Switch 3 (Water Pump): Manual at upper position in which water pump will turn ON directly if the “valves and pumps” breaker is ON and the switch is in its upper position. Otherwise, it will turn ON/OFF according to controller command if switched to lower position.

- Switch 4 (KoH Pump): Manual at upper position in which KoH pump will turn ON directly if the “valves and pumps” breaker is ON and the switch is in its upper position. Otherwise, it will turn ON/OFF according to controller command if switched to lower position.

- Switch 5 (Water Valve): Manual at upper position in which water pump will turn ON directly if the 12 V power adapter is ON and the switch is in its upper position. Otherwise, it will turn ON/OFF according to controller command if switched to lower position.

- Controller: The controller used is Arduino Mega. A nrf24l01 radio frequency antenna is used to establish wireless communication between the plant and the remote station. It is used to control the relay box according to operator’s command (from the user-end GUI) and send back the KoH and water levels (Low – Medium – High).

- Level Detector: A circuit that detects the current liquid level (Low – Medium – High).

- Relay Box: Controlled contacts (8 channels).

## Wiring Diagram

The wiring diagram should follow the safety standards and appropriate cable sizing to ensure maximum operating efficiency. The size of wires was selected upon the load required power. The below table shows the selected cable sizes with respect to every load consumption.

| Load | Current Consumption (A) | Wire Size (mm2) |
| --- | --- | --- |
| Cell Power Source | ~10 | 2.5 |
| Cell Valves | 0.145 | 1.5 |
| Water Pump | Type equation here. | 1.5 |
| KoH Pump | Type equation here. | 1.5 |
| Water Return Valve (DC) | Type equation here. | 1 |
| Automatic Valves (DC) | 0.35 | 1 |

The below figure illustrates the power system connections. The relay box is controlled through Arduino’s digital pins according to a command received (by nrf24l01 module) from the user GUI. The Switches (SW1 to SW5) select the mod of operation. The Controller is responsible for sending back the O2 and H2 liquid levels to the user GUI through its transceiver module (nrf24l01). NOTE: SW1 is MALFUNCTIONED and is not used at the moment!

## Level Detector

The level detector is an electronic circuit designed to monitor the current available amount of O2 and H2 levels in the containers. The circuit outputs 6 digital data signals representing the current liquid level (3 for each container: Low – Med – High) compatible with the Arduino Controller (@ 5V).

### Schematics and Principle of Operation

The above 2 figures illustrate the soldered board and the circuit schematic respectively. As can be seen from the schematic, each container includes 4 wires with different lengths and 3 transistor switches (each represents a certain state; Low – Med – High) whereas the 4th wire carries the positive 5V from the Arduino. The collectors of the transistors are connected to the Arduino’s digital pins as input which are naturally pulled HIGH (5V). When liquid is present in the container, a short circuit is established between the 5V wire and the level wires (Low – Med – High) which in turn will bias the transistor and switch it ON. When a certain transistor is ON, the specific Arduino pin is pulled LOW (0 V). The below code is used to test the liquid level detector circuit.

## Flow Chart and Source Code

The controller of the electrolyzer plant will send the O2 and H2 levels to the user GUI every 200 ms. The RF module (nrf24l01) will switch its role every 200 ms from receiving to sending. NOTE: The below 2 codes should be in the same folder in order for them to work.

The source code (text) is shown below:

/*FOR ARDUINO MEGA: MASTER ADDRESS*/

#include <SPI.h>

#include <Wire.h>

#include <nRF24L01.h>

#include <RF24.h>

#include<Servo.h>

#define cePin 3

#define csnPin 4

RF24 radio(cePin, csnPin); // Create a Radio

//Servo av_oxygen, av_hydrogen;

// RADIO VARIABLES //

const byte slaveAddress[5] = {'R', 'x', 'A', 'A', 'A'};

const byte masterAddress[5] = {'T', 'X', 'a', 'a', 'a'};

char dataToSend[10] = "Message 0";

char txNum = '0';

int dataReceived[2]; // to hold the data from the slave - must match replyData[] in the slave

bool newData = false;

unsigned long currentMillis;

unsigned long prevMillis;

unsigned long txIntervalMillis = 200; // send once per 200 ms

// RELAY PINS //

byte relay_pins[5] = {13, 12, 11, 10, 9};

int tx_failed_count = 0;

boolean relay_status[5] = {0, 0, 0, 0, 0};

// LIQUID LEVEL //

byte oxygenLevel_pins[3] = {22, 24, 26}; // LOW - MED - HIGH

byte hydrogenLevel_pins[3] = {28, 30, 32}; // LOW - MED - HIGH

byte water_hyd_level[6];

void setup() {

Serial.begin(9600);

radio_init();

for (int i = relay_pins[0] ; i > 8; i--) pinMode(i, OUTPUT);

for (int i = oxygenLevel_pins[0]; i < hydrogenLevel_pins[2] + 1; i += 2) pinMode(i, INPUT);

for (int i = 0; i < 4; i++) digitalWrite(relay_pins[i], 1);

}

void loop() {

for (int i = 0; i < 3; i++) {

water_hyd_level[i] = 1; //!digitalRead(oxygenLevel_pins[i])

water_hyd_level[i + 3] = 0;//!digitalRead(hydrogenLevel_pins[i]);

}

currentMillis = millis();

if (currentMillis - prevMillis >= txIntervalMillis) {

send();

prevMillis = millis();

}

getData();

showData();

for (int i = 0; i < 5; i++)

digitalWrite(relay_pins[i], !relay_status[i]);

}

void radio_init() {

radio.begin();

radio.setDataRate( RF24_250KBPS );

radio.openWritingPipe(slaveAddress);

radio.openReadingPipe(1, masterAddress);

radio.setRetries(3, 5); // delay, count

send(); // to get things started

prevMillis = millis(); // set clock

}

// Will send Liquid level (H2O - Hyd) levels to the station @ 500 ms

void send() {

radio.stopListening();

bool rslt;

rslt = radio.write( &water_hyd_level, sizeof(water_hyd_level) );

radio.startListening();

Serial.print("Data Sent (Levels) ");

for (int i = 0; i < 6; i++)

Serial.print(String(water_hyd_level[i]) + " ");

Serial.println();

if (rslt) {

Serial.println("  Acknowledge received");

updateMessage();

}

else {

Serial.println("  Tx failed");

tx_failed_count++;

// If the transmission failed for 10 times

if (tx_failed_count == 10) {

// Turn OFF ALL Relays

for (int i = 0; i < 5; i++)

relay_status[i] = 0;

tx_failed_count = 0;

}

}

Serial.println();

}

// Will receive relay status from the station @ 500 ms

void getData() {

if ( radio.available() ) {

radio.read( &relay_status, sizeof(relay_status) );

newData = true;

}

}

void showData() {

if (newData == true) {

Serial.print("Data received Relay Status ");

for (int i = 0; i < 4; i++)

Serial.print(String(relay_status[i]) + " ");

Serial.println();

newData = false;

}

}

// JUST FOR TESTS! IS NOT PART OF THE CODE

void updateMessage() {

// so you can see that new data is being sent

txNum += 1;

if (txNum > '9') {

txNum = '0';

}

dataToSend[8] = txNum;

}

# User-End

This section discusses the interaction between the user GUI and the Arduino Uno controller which is used to receive relay status data from the GUI and send them to the electrolyzer controller.

## GUI

The GUI interface is shown in the image below. It is divided into 3 sections; The Power and connect, monitoring, and control.

Available COM Ports

- Connect Button: This button is used to connect to the Arduino controller. It uses the COM port shown in the COM port combo box below it. All buttons will not work until the computer is connected to the station Arduino.

- Cell Power Button: if the electrolyzer is set to Auto mode and this button is pressed, the cell power (welding machine) is activated and its indicator (red box next to it) should turn green.

- The H2 and O2 Bars: Will indicate the current amount of liquid found in the condenser.

- Automatic valves (1 and 2): These sliders will control the servo valves at the indicated angle label (number to the left and right).

- Water Pump, KoH Pump, Water Return Valves, and Cell Valves: These buttons will control the respective appliances.

## Controller

When Data come from the GUI, the “Serial Event” event handler function is stimulated. This function will read the incoming string sent from the GUI and will take the decision accordingly. The codes of the user station are attached below.  NOTE: The below 2 codes should be in the same folder in order for them to work.

The Arduino code (in Text) is shown below:

/*FOR ARDUINO UNO: SLAVE ADDRESS*/

#include <SPI.h>

#include <Wire.h>

#include <nRF24L01.h>

#include <RF24.h>

#define cePin 7

#define csnPin 8

RF24 radio(cePin, csnPin); // Create a Radio

// FOR GUI CONFIG //

String inputString = "";         // a string to hold incoming data

boolean stringComplete = false, ledState = false;  // whether the string is complete

String commandString = "";

boolean isStarted = false;

boolean dataToSend[11];

int autoValve;

// RADIO VARIABLES //

const byte slaveAddress[5] = {'R', 'x', 'A', 'A', 'A'};

const byte masterAddress[5] = {'T', 'X', 'a', 'a', 'a'};

bool newData = false, txDone;

unsigned long currentMillis;

unsigned long prevMillis;

unsigned long txIntervalMillis = 200; // send once per 200 ms

// CONTROL ACTIONS //

boolean oxy_hyd_levels[6] = {1, 0, 0, 1, 0, 0};

boolean relay_status[5] = {0, 0, 0, 0, 0};

int oxy_hyd_levels_size = sizeof(oxy_hyd_levels) / sizeof(oxy_hyd_levels[0]);

int relayStatus_size = sizeof(relay_status) / sizeof(relay_status[0]);

const int dataToSend_size = oxy_hyd_levels_size + relayStatus_size;

void setup() {

Serial.begin(9600);

radio_init();

}

void loop() {

// Data Received From GUI

if (stringComplete)  {

stringComplete = false;

int commandLength = commandString.length();

commandString.remove(0, 1);

commandString.remove(commandLength - 1);

if (commandString.equals("STAR")) isStarted = true;

else if (commandString.equals("STOP")) isStarted = false;

else if (commandString.equals("1")) relay_status[0] = !relay_status[0];

else if (commandString.equals("2")) relay_status[1] = !relay_status[1];

else if (commandString.equals("3")) relay_status[2] = !relay_status[2];

else if (commandString.equals("4")) relay_status[3] = !relay_status[3];

else if (commandString.equals("5")) relay_status[4] = !relay_status[4];

else autoValve = commandString.toInt();

}

// Data to send back to GUI

if (isStarted) {

String inString = "";

for (int i = 0; i < dataToSend_size; i++)  {

String sep = ",";

if (i == dataToSend_size - 1) sep = "";

if (i < relayStatus_size) dataToSend[i] = relay_status[i];

else dataToSend[i] = oxy_hyd_levels[i - relayStatus_size];

inString += String(dataToSend[i]);

//dataToSend = {R1, R2, R3, R4, R5, O2-L, O2-M, O2-H, H2-L, H2-M, H2-H}

//Serial.println(String(dataToSend[i]) + sep);

}

Serial.print(inString+ "\n");

}

// Control of Auto Valves 1 and 2

if (autoValve > 349 && autoValve < 1351) {

// Code for auto valve 1

int autoValve1_angle = autoValve / 10;

}

else if (autoValve > 3000) {

// Code for auto valve 2

int autoValve2_angle = autoValve / 100;

}

// RF FUNCTIONS //

// Get the liquid levels from the plant

getData();

// Send the relay status to the plant

send();

delay(10);

}

void serialEvent() {

while (Serial.available()) {

commandString = Serial.readStringUntil('\n');

}

stringComplete = true;

}

void radio_init() {

radio.begin();

radio.setDataRate( RF24_250KBPS );

radio.openWritingPipe(masterAddress); // NB these are swapped compared to the master

radio.openReadingPipe(1, slaveAddress);

radio.setRetries(3, 5); // delay, count

radio.startListening();

}

// Will send relay status to plant

void send() {

if (newData == true) {

radio.stopListening();

bool rslt;

rslt = radio.write(&relay_status, sizeof(relay_status) );

radio.startListening();

if (rslt) txDone = true;

else txDone = false;

newData = false;

}

}

// Will work every txIntervalMillis @ Plant Side

void getData() {

if ( radio.available() ) {

radio.read( &oxy_hyd_levels, sizeof(oxy_hyd_levels) );

newData = true;

}

}