North Lebanon Automotive Systems

ANNUAL REPORT 2024

- Investments

- Production of a Electrical 4-wheel transporter Tuk-tuk with solar system and Lithium Batteries

- Building of a Production Facility in Ras Nhache

Author:

Samir Mourad

Last Update: 19.12.2024 20:28

Table of Contents

Preface	1

1	Planning&Controlling 2024	2

2	Introduction	3

3	Investments	4

4	E-Transporter Summary	5

5	Technical Issues from Development of E-Transporter Prototype 2022-2023	7

5.1	System Design	7

5.2	E-Tuktuk Mechanical Design	8

5.2.1	Chassis	8

5.2.2	Chassis design (Oct 2023)	11

5.2.3	Front axis	20

5.2.4	Front Wheels	20

5.2.5	Rear Wheels	32

5.2.6	Excel sheet for all vehicle components:	32

5.3	Enhancement Report (Oct-Dec 2023)	33

5.3.1	New chassis design	33

5.3.2	New roof design	35

5.3.3	Chassis and roof quotations (2 quotations min)	36

5.3.4	Vehicle components list  (with an image notes each component on the vehicle)	37

5.3.5	Components quotations (2 quotations min)	37

5.3.6	Electrical study	38

5.3.7	Official answers	38

5.4	Electric/Electronic of E-Tuktuk	39

5.4.1	Parts	39

5.5	Lithium-Ion Batteries Manufactoring Concept and Battery Management System (BMS) Design	49

5.5.1	Overview	49

5.5.2	Lithium Battery Prototype Manufactoring	51

5.5.3	Lithium-Ion Battery Charger	51

5.6	Agricultural Accessories	65

5.6.1	Mechanics of Agricultural Acessories	65

5.6.2	E/E of Agricultural Acessories	67

5.7	Steps of work	69

5.7.1	Assemblage the Tuk-Tuk (10/1/2023)	69

5.7.2	Installing a new structure for solar panels (25/1/2023)	69

5.7.3	Small Adjustments (1/2/2023)	69

5.7.4	Modified basic mechanics: (8/2/2023)	70

5.7.5	Transfer the red tuk-tuk to Ras Nhache (25/2/2023)	72

5.7.6	Trying the little blue dibble on the red tuk-tuk (12/3/2023)	72

5.7.7	Work on the crane of the dibble for red and blue tuk-tuk’s (25/3/2023)	73

5.7.8	Installing the big red Dibble on the blue Tuk-tuk. (15/4/2023)	74

5.7.9	Installing the big red Dibble on the red Tuk-tuk. With modification to the crane base (17/4/2023)	74

5.7.10	Testing the big Dibble (23/5/2023)	74

5.7.11	Adjusting the crane base on the red tuktuk (30/5/202)	75

5.7.12	Paint the Tuk-tuk red (1/6/2023)	75

5.7.13	Testing the Tuk-tuk (6/6/2023)	75

5.7.14	Receiving the store next to the center and transporting the Tuk-tuk to it from Ras Nhach.(10/6/2023)	76

5.7.15	Design and installation of an irrigation system on a red Tuk-tuk. With some modifications from the old system in the blue Tuk-tuk (22/6/2023)	76

5.7.16	Grass shredder installation. (3/7/2023)	76

5.7.17	Installing a 12V water pump on the red Tuk-tuk (7/7/2023)	77

5.7.18	Irrigation test (10/7/2023)	77

5.7.19	20- Installing two front tires instead of one tire on the red tuk-tuk.     (16/8/2023)	77

5.7.20	Testing and Marketing the red Tuk-tuk (20/8/2023)	78

5.7.21	The new Tuktuk's front budget is painted red (31/8/2023)	78

5.7.22	Used the red tuk-tuk by AECENAR team(Power plan) (20/8/2023 to present)	79

6	Proposal for Investment in Sustainable Electric Car Project	81

6.1	Executive Summary	81

6.2	Project Overview	81

6.3	Business plan	82

6.3.1	Partnership Duration	82

6.3.2	Business Model Partnership:	82

6.3.3	Pricing Strategy:	82

6.3.4	Responsibility Matrix	83

6.4	4. Financials	83

6.4.1	Budget estimation	83

6.5	5. Conclusion and Call to Action	85

6.6	ANNEX I : Product Description	86

6.6.1	Vehicle Specification	86

6.6.2	Solar Integration:	86

6.6.3	Lithium Battery:	86

6.6.4	Safety Measures:	86

6.6.5	Customization Options:	86

6.7	ANNEX II : Market study - Maintenance and operations comparison and cost saving	87

6.7.1	Maintenance Expenses:	87

6.7.2	Fuel Expenses	87

6.8	ANNEX III : Responsibility Matrix table	87

6.9	Annex IV : Phase Budget Estimation	88

6.9.1	Phase 1: Cargo Vehicle	88

6.9.2	Phase 2: Passenger Vehicle	88

6.9.3	ANNEX V : Ready to Market Time Plan	89

7	Production Facility	90

7.1	Production Line	90

7.1.1	Costs Excel table:	91

7.1.2	Production Line (PL) development:	91

7.1.3	Manufactoring Manual & Production Timeline	93

7.2	Building of Facility in Ras Nhache	97

8	Mafrak – Ras Nhache line	98

8.1	Business Specifications	98

8.2	Design of Persons Transport	98

Reference	99

Preface

This report contains details of our implementation of the 2023 NLAS project. The presented project is 4 Wheel E-TRACTUK Electric Tractor based on Tuk-Tuk chassis.

# Planning&Controlling 2024

From Tuk-tuk Enhancement Oct-Dec 2023:

Planning Tuk-Tuk Enhancement 2 (Dec 23):

# Introduction

We have several positive aspects of this project:

- Mechanically: The tuk-tuk with solar cells has good driving power and is easy to maintain. They were able to achieve more durability after new mechanical modifications. After changing to 4 Wheel tuktuk we have more safety and grip.

- Electrically: Simply charge the battery and check the battery acid level and it works without motor, heat and noise.

- Health: Without any pollution, which provides a clean environment.

- Practically: The transportation is comfortable with more features for your comfort. You have more space for the driver than a traditional tuk-tuk. The way it rotates makes it easier to control as well.

# Investments

# E-Transporter Summary

E-Rapid Report

Completing the journey of development and enhancement of the E-rapid to reach an innovative product.

1. Introduction:

The E-Rapid is an Electric Vehicle that is meant to transport goods within cities. This E-rapid is a rear-wheel-drive vehicle that is fully electric, it is powered by an advanced solar system that assists the vehicle to charge.

1.1. Brief Timeline recap of the development of E-transporter:

- Assembly of E-tuktuk

- Designing and manufacturing of solar panel system

- Installing new front suspension with two wheels

Fig. 1. E-transporter final design [1]

1.2. Current specs of the E-transporter

- Batteries a series of five 12V Gel batteries sums a 60V cell of 45.2 Ah.

- Electric motor a brushless DC motor (BLDC) of rated voltage 60V, power 1000W and 3000 rpm.

- Controller offers reversing, 3 speed shift, over temperature protection.

- Solar system consists of 5 solar panels of 100W with an inverter that is directly connected the charge the batteries when possible.

2. Recommendations for the final E-rapid product:

- Improve front suspension weldments.

- Replace the oil wheels (smoother threads)

- Install Li-ion batteries and increase their capacity

- Improve solar system installation (wires and inverter)

2.1. Improvements descriptions:

- Front suspension that was added to the vehicle to maintain 2 wheels, has loose connections and weldments. To increase its quality and performance, advanced FEM simulations should be drawn out to check its viability, it may need some optimizations in its design.

- Current wheels are the agricultural use, for asphalt use smoother thread wheels to decrease loses.

- Wires and the inverter of the solar system can be packed in a better and safer way.

- Installing Li-ion batteries is a just winning step since the vehicle will be lighter and has batter range.

Mass rough estimation of Li-ion batteries according to its energy density:

Assumption if the lithium batteries will have the same specs the current gel battery of 60 V and 45.2 Ah.

Lithium-ion batteries energy density is between 200-300 Wh/kg [2]. The energy of the battery is its voltage by capacity that implies 2700 Wh. The mass of the battery is its energy over materials energy density, the minimum energy density will be used of 200 Wh/kg. The rough estimated mass is 15 kg, for sure that an error margin depends on the material used.

Mass=Energy DensityEnergy​=200Whkg2700Wh​=15kg                                                                    (1)

According to this mass gain we can increase the capacity/range.

3. conclusion

Optimizing the current E-transporter to reach the ideal stage of performance and quality takes our product to the market-ready status. This includes the enhancements of the key variables like range, battery efficiency, drivetrain performance and overall satisfaction. By doing so, we can confidently introduce to the market our E-Rapid

References

1. NLAS ANNUAL REPORT 2023

2. Energy, T. S. (2023, July 14). Lithium ion batteries: energy density?

# Technical Issues from Development of E-Transporter Prototype 2022-2023

## System Design

We radically modified the design on the front side, after conducting several experiments on the previous version of Tuk-tuk. We found a problem with balance when driving, and after checking we discovered that it was due to the front wheel. So, we replaced the old one-wheel budget, with a two-wheel budget ATV, with some modifications to fit the Tuk-Tuk chassis.

## E-Tuktuk Mechanical Design

### Chassis

#### E-TukTuk FreeCAD Drawing

Tbd: Chassis 2D drawings

17-11-2022:

E-TukTuk FreeCAD Drawing 			
E-TukTuk FreeCAD 24-10-22

#### Mechanical Realization

### Chassis design (Oct 2023)

#### chassis design(oct2023)

FreeCAD design of new chassis (21.10.23) :

##### FreeCAD design 7-11-2023_new chassis with front suspension:

##### FreeCAD design for solar panels stand:

31-10-2023_solar panels stand new :

#### Assembly of new chassis and solar panels stand (Version Oct 2023)

31-10-2023_new assembly chassis with front suspension and solar panels:

##### 2D Parts Drawings:

###### Extension link from front suspension to main chassis

FreeCAD 2d and 3d drawing of the extension link between front suspension and main chassis :

###### 2- Front main chassis:

2D and 3D drawing and techdraw for the front main chassis design :

###### Back main chassis:

2d and 3d drawing of the back part of the main chassis :

##### Excel sheet for all vehicle components

Excel einfügen

31-10-2023_vehicle components list

#### Chassis Version 7 Nov 2023

The new chassis and solar panel stand costs 500$ including material and labor work fees

FreeCAD design 3-1-24_NEW tuktuk chassis with solar panel stand

FreeCAD design for solar panels stand:

31-10-2023_solar panels stand new

### Front axis

#### FreeCAD Drawing

3D tbd

2D tbd

#### Mechanical Realization

tbd

### Front Wheels

#### FreeCAD Drawing

3D tbd

2D tbd

#### Mechanical Realization

tbd

#### Front wheel rim disc

##### design:

17-22-2022 drawing:

28-11-22 E_TukTuk with solar panel stand:

30-11-22 E_TukTuk with solar panel stand and hitch handle:

5-12-22 E_TukTuk:

17-12-22 :

19-12-22 E_TukTuk with solar panel stand:

| 28-11-22 E_TukTuk with solar panel stand: | 5-12-22 E_TukTuk |
| --- | --- |

| 17-12-2022 New Solar Panels stand | 17-12-2022 E-tukTuk |
| --- | --- |

| Hitch Design | 19-12-22 New Hitch design |
| --- | --- |

##### Realization

### Rear Wheels

#### FreeCAD Drawing

3D tbd

2D tbd

#### Mechanical Realization

tbd

### Excel sheet for all vehicle components:

31-10-2023_vehicle components list:

## Enhancement Report (Oct-Dec 2023)

| Task | Deliverables |
| --- | --- |
| Design new chassis | CAD design |
| Design & enhance the vehicle roof with the solar panel | CAD design |
| Find a manufactory for the chassis and solar roof | Quotations and contacts |
| Specify all vehicle components list | Excel sheet |
| Find provider for each component | Quotations and contacts |
| Fully check the battery requirement and solar charging | Electrical study |
| Start preparing for the license agreement | Official Lebanese answers |
| Documentation (Always UpToDate ) | Word file and Website |
| Weekly meeting | Online/onsite weekly meeting |

The assembled design of the chassis and solar panel roof design:

### New chassis design

#### Chassis CAD file:

#### Chassis design screenshot:

#### Needed 2D detailed designs:

### New roof design

#### roof CAD file

#### roof design screenshot:

#### Needed 2D detailed designs:

### Chassis and roof quotations (2 quotations min)

No official quotation sheet,

- Abo Al Abeds quotation : 500$

- Khaled Saleh CNC : 750$ if we order one peace, another price for a 5 plus chassis order.

### Vehicle components list 
(with an image notes each component on the vehicle)

### Components quotations (2 quotations min)

Al halabi is the only local supplier of electric tuktuk parts with best prices and prices less then AliBaBa in the term of small orders

### Electrical study

Abdallah Kassem

### Official answers

محمد الصعيدي مخلص معاملات

+961 70 113 677

## Electric/Electronic of E-Tuktuk

### Parts

#### Controller

#### 1000w Electric Motor:

#### The Throttle Handlebar and lighting/Flasher controller:

#### Lighting and flashers

#### 5 Batteries 12V , 45.2 Ah

#### Batteries charger

#### All electric parts installation with batteries and controller

## Lithium-Ion Batteries Manufactoring Concept and Battery Management System (BMS) Design

### Overview

### Lithium Battery Prototype Manufactoring

### Lithium-Ion Battery Charger

This is Lithium-Ion battery charger implemented on Arduino. Has more advanced features like:

- State of charge estimation.

- EEPROM logging.

- Command-Line interface.

It uses the constant current constant voltage (CC-CV).

The rationale behind this project was to upgrade the depleted battery pack and charger of an old cordless drill from Nickel-Cadmium (Ni-Cd) to Lithium-Ion (Li-Ion) technology.

Warning: Lithium-ion batteries are dangerous devices. Overcharging, short circuiting, or misuse of lithium-ion batteries may result in fire and/or violent explosion. It is necessary to equip each lithium-ion battery with its own dedicated battery protection board (or battery management system also known as BMS).

#### Theory of Operation

The following subsections cover the theoretical and mathematical aspects of charging a Li-Ion battery.

##### CC-CV Charging

Li-Ion batteries must be charged using the Constant Current Constant Voltage (CC-CV) charging method. This method consists of charging the battery at a constant current Icharge until a certain voltage threshold Vmax=4.2Vcell is reached, then gradually reducing the charging such that the constant cell voltage Vmax is not exceeded. Charging is terminated once the current reaches a certain minimum threshold Ifull of typically 50-150 mA.

Additional End of Charge (EOC) standards have been implemented for safety reasons. These include time-based and capacity-based EOC detection. When the battery is connected, the charger measures the voltage at its terminals. The SOC value is used to calculate the remaining capacity Cmax and charging duration Tmax. Charging is terminated if any of these values ​​are reached.

##### Control Loop

The battery “+” terminal is connected to the positive power supply through a power MOSFET (field-effect transistor). The battery “-” terminal is connected to the power supply ground through a low-value shunt resistor Rshunt.

The charging current is regulated by pulse width modulation (PWM), where the MOSFET is periodically turned on and off by the Arduino at a frequency of 31,250 kHz. The charging current is controlled by gradually adjusting the PWM duty cycle which is the ratio between the ON and OFF duration of the MOSFET.

V1 is the voltage measured at the “+” terminal of the battery and V2 is the voltage measured at the “-” terminal of the battery. Both voltages are measured relative to the power supply ground and are used to calculate the voltage V across the battery pack and the charging current I as follows:

VVolt=V1-V2

I(Ampere)=V2Rshunt

Two separate ADC channels on the Arduino are used for measuring the above voltages. The Arduino continuously monitors V and I and adjusts the PWM duty cycle in order to achieve the desired constant current or constant voltage regulation.

##### State-of-Charge Estimation

The state of charge SOC is estimated by reading the battery voltage V and comparing it to a series of values stored in a lookup table L = (l0, l1, l2, l3, l4, l5, l6, l7, l8). The threshold voltages are derived from the particular discharge curve shown below for the LG 18650 HE4 cells used in this project. (Source: https://lygte-info.dk/review/batteries2012/LG%2018650%20HE4%202500mAh%20%28Yellow%29%20UK.html).

Figure 1: Statistic of Discharge, capacity (LG-18650-HE4-2500mAh)

The red discharge curve corresponding to 0.2A discharge current has been used, whereas the values of L were assigned such that:

- Ɩ0= V~2.25Ah

- Ɩ1= V~2.00Ah

- Ɩ2= V~1.75Ah

.

.

- Ɩ8= V~0.25Ah

SOC is calculated as follows:

- V< Ɩ0: SOC= 0%

- Ɩ0<V< Ɩ1: SOC= 10%

- Ɩ1<V< Ɩ2: SOC= 20%

.

.

- Ɩ8<V: SOC= 90%

The remaining capacity Cmax and charge duration Tmax are derived as follows:

Cmax (mAh)=Cfull ×(100-SOC)×1.3

Tmax=3600×Cfull Icharge  × (90 – SOC) + 45.6

Where C full is the design capacity of the battery and C is the nominal charging current. Note that Cmax is increased by 30% and Tmax is increased by 45 min in order to account for resistance losses and inaccuracy of SOC estimation.

##### Safety

The charger implements several safety features. These include:

- Undervoltage.

- Overvoltage.

- Short circuit.

- Open circuit detection.

The typical voltage range where a Li-Ion battery can safely operate is between Vmin = SI2.5Vcell  and Vmax = SI4.2Vcell  . Operating outside this range is likely to cause permanent damage to the Li-Ion cells and may even result in a catastrophic failure such as an explosion or fire. In addition, the battery pack is protected by a battery protection board (or battery management system also known as BMS). The BMS measures the voltages of individual battery cells as well as the charging and discharging current flowing through the battery. The BMS uses a solid-state switch to disconnect the battery once the voltage or current values ​​become outside the specified limits. For the most part, the BMS is completely transparent and does not interfere with the charging process, except for the case where the BMS disconnects the depleted battery in order to prevent over-discharge. In this case, the voltage of the depleted battery is still present across the BMS terminals through a high value resistor placed in series with the battery. This high value resistor causes a much lower voltage value to be measured at the charger terminals. Consequently, the charger must ignore the Vmin lower limit and start charging at a much lower value of as low as Vstart = SI0.5Vcell . When served with a depleted battery, the charger will start charging at a low safety current Isafe =Icharge 10  until the battery voltage reaches Vsafe = SI2.8Vcell , after which full charging current Icharge will be applied. Once the voltage reaches this threshold, it is no longer allowed to drop below Vmin. A voltage lower than Vmin may cause an “under voltage fault” which may be caused by either a short circuit or open circuit of the battery. Open circuit is also detected if the charging current stays equal to zero while the PWM duty cycle increases beyond a specific threshold. This condition would raise an “open circuit error”. Overvoltage is detected whenever the battery pack voltage momentarily exceeds Vsurge = SI4.25Vcell . Exceeding this value would raise an “overvolt error”.

##### Trickle Charging

Once the end-of-charge (EOC) criteria has been met, the charger would cut-off the charging current and switch to an idle mode where it will continuously monitor the battery voltage. Once the voltage drops below a specific threshold of Vtrickel/start = SI4.10Vcell , a new charging cycle will be initiated using the following parameters:

Vmax (Vcell )=Vtrickel_max=(4.15)Vcell

Cmax(mAh) =Cfull×0.3+C

Tmax(s)=20×60+T

Where Cfull is the battery design capacity. C and T are the accumulated charge capacity and charge time since the battery has been connected, including the initial charge and all of the subsequent trickle charge cycles. Given the above formulas, the trickle charge cycle uses a reduced Vmax and allows for charging up to a maximum of 3% of the battery design capacity during a maximum duration of 20 minutes.

#### Hardware

The following sub-sections describe the hardware design aspects of the Li-Ion charger.

##### Mechanical Design

We used four LG 18650 HE4 Li-Ion cells and a battery protection board (or battery management system also known as BMS). Modern lithium-ion cells use much less space.

##### Battery Protection Board

It is necessary to use a dedicated battery protection plate for each battery pack. This provides an extra layer of protection to prevent over charging or over discharging due to software or hardware malfunction. In figure 2 below, it shows the 4S/30A (4S means 4 cells in series) battery protection board (or BMS).

Figure 2: 4S Battery Protection Board

In figure 2 can see the wiring diagram for connection the 4 Li-Ion cells with the BMS.

This particular BMS includes the cell balancer feature. If the voltage of one or more cells becomes higher than the rest of the pack, the BMS would actively discharge those cells to ensure that all the cells of the battery pack share the exact same voltage.

##### Circuit Diagram

In figure 3 below, it shows the Li-Ion charger circuit diagram.

Figure 3: Li-Ion charger circuit diagram

In figure 3 above schematic, the 19.5 V of the power supply are stepped-down to 5 V by the 7805-voltage regulator U1. The 5 V is used for powering the Arduino board.

The Arduino Pro Mini compatible board U2 hosts an AT-mega 328P microcontroller running at 16 MHz clock frequency and is used as the main processing unit for the device.

The Lithium-Ion battery is connected across the B+ and B- terminals. The battery charging current is regulated by switching P-Channel MOSFET (field-effect transistor) Q1 via pulse-width modulation (PWM).

The PWM-enabled digital output pin 9 on the Arduino generates a PWM signal which drives the gate of the MOSFET Q1 through the NPN transistor Q2. The voltage divider formed by R1 and R2 ensures that the gate-source voltage of the MOSFET stays within the specified limits.

A current-sensing shunt resistor connects the B- terminal with ground. It consists of two 1 Ω / 3 W resistors R8 and R9 connected in parallel. This results in a total resistance of 0.5 Ω. At a charging current of Icharge=2A, the voltage across the shunt will be exactly 1 V; which is slightly below the 1.1 V internal voltage reference of the Arduino thus corresponds to the full range of the Arduino’s analog-to-digital converter (ADC).

The analog pin A0 on the Arduino is used for measuring the voltage V1 between B+ and 0 V. and the Analog pin A1 is used for measuring V2 between B- and 0V.

B+ is connected to pin A0 through a voltage divider consisting of R4 and R7, the ratio has been chosen such that the maximum battery pack voltage of 16.8 V would result in slightly less than the Arduino’s internal reference voltage of 1.1 V at A0. Please note that the value of R4 needs to be adapted to the number of cells in use. For example, using a 1 cell setup would require reducing the value of R4 to 39 KΩ.

B- is connected to A1 through a current-limiting resistor R5; A voltage divider is not required for measuring V2 as its value stays below the Arduino’s ADC internal reference voltage.

Two 100 nF capacitors C4 and C5 are used for blocking the high-frequency noise caused by the PWM from reaching the analog inputs, an essential measure for smooth ADC readings.

The Diode D1 protects the 7805 regulator from a reverse power supply polarity. The diode D2 protects the battery from a reverse polarity. it also prevents the battery from feeding power back into the Arduino in case the main power supply has been disconnected.

A LED indicator D3 and its dropper resistor R6 are connected to Arduino’s digital pin13.

Important: The battery terminals in the circuit diagram are labeled as B+ and B-. It is important to connect these terminals to the P+ and P- terminals of the Battery Management System (BMS) depicted in the figure 3. The BMS has its own set of B+ and B- terminals that must be connected directly to the battery terminals. It is crucial to avoid connecting the charger’s B+ and B- terminals to the B+ and B- terminals of the BMS, as this would bypass the BMS and prevent it from safeguarding the battery against overcharging.

##### Different Number of Cells

The following values for R2, R4 and the power supply voltage need to be chosen in order to charge different numbers of Cells:

| Ncells | Power Supply | R2 | R4 |
| --- | --- | --- | --- |
| 1 | 5V-6V | 220Ω | 39KΩ |
| 2 | 10V-15V | 100Ω | 82KΩ |
| 3 | 14V-20V | 220Ω | 120KΩ |
| 4 | 18.5V-20V | 220Ω | 180KΩ |

Table 1: Ncell with Power Supply, R2, R4

When charging 1 cell, the following circuit modifications must be performed:

- Remove the voltage regulator U1 and capacitor C3 and power the Arduino directly from the output of D1.

- Replace Q1 with a IRLML2244 MOSFET.

- Increase R1 to 10 KΩ.

- Remove Q2 and R3.

- Connect R2 directly to Arduino digital pin 9.

- Modify the code in li-charger.ino to invert the PWM signal by subtracting the PWM duty cycle from 255 within all instances of analogWrite () using one of the following statements in the figure below:

Figure 4: code of analogWrite () in li-charger.ino

#### PCB Layout

All components are of the punch-hole type and are mounted on a PCB board. The Figure 4 shows the PCB layout of a Li-Ion charger.

Figure 5:PCB layout of the Li-Ion charger

The MOSFET Q1 (TO-220 device in the top right corner) and large green-colored shunt resistors will get pretty hot so adequate ventilation needs to be assured. The following measures has been taken to avoid overheating:

- The shunt resistors R8 and R9 are raised by around 5mm from the PCB in order to assure adequate cooling.

- A series of holes has been drilled in the bottom of the enclosure in order to allow for a better air flow.

- The charging current Icharge has been limited to 1.5 A.

The electrolytic capacitor C1 towards the top center of the board is in a sub-optimal position due to its location between two hot components, the 7805 regulator and the MOSFET. High temperatures reduce the lifespan of electrolytic capacitors thus the must be kept away from heat sources.

The pin header located at the top right corner is used for connecting all the external wires. Following is the pinout assuming that pin 1 is at the top right corner and pin 10 is towards the middle of the board.

| Pin | Purpose |
| --- | --- |
| 1 * | LED + |

| 2 * | LED - |
| --- | --- |

| 3,4 ⸸ | Power supply + |
| --- | --- |

| 5,6 ⸸ | Battery + |
| --- | --- |

| 7,8 ⸸ | Power supply - |
| --- | --- |

| 9,10 ⸸ | Battery - |
| --- | --- |

Table 2: All pinhead work

* The LED dropper resistor is located on a separate PCB together with the LED itself.

⸸ Two pins are connected in parallel in order to increase their current capacity.

#### User Interface

The following sections describe the user interface of the Lithium-Ion charger. It consists of a LED indicator and a Command-Line Interface (CLI).

##### LED Indicator

The charger status is displayed by turning on or flashing a single LED as shown

in Table 3.

| Blinking Pattern | Meaning |
| --- | --- |
| On for half a second every 2 seconds | Ready, waiting for the battery to be connected |
| Solid on | Battery charging |
| On For 0.1 second every 2 seconds | Battery fully charged |
| Blinking fast (0.4 s period) | Error |
| Blinking very fast (0.2 s period) | Calibration mode |

Table 3: The meaning of what is displayed via the LED

##### Command-Line Interface

This Lithium-Ion battery charger features a Command-Line Interface (CLI) that can be accessed via the Arduino’s RS232 serial port. The easiest way to connect to the CLI is to open the serial monitor of the Arduino IDE while connected to the charger using a FTDI USB to Serial converter. Please ensure that the Baud rate is set to 115200.

Once up and running, the charger will display a welcome message on the serial monitor, show the current firmware version and present with the list of available commands shown in the following list.

Some of these CLI commands need to be provided with arguments. Thus, one needs to enter the command followed by one or two arguments separated by a white space. SSS

| Command | Description |
| --- | --- |
| h | Help- show the list of available commands |

| . | Display the real-time parameters, including the charge duration T, charge capacity C, battery voltage V, charging current I, maximum charge duration Tmax, maximum charge capacity Cmax, maximum charging voltage Vmax, maximum charging current Imax, PWM duty cycle, voltages V1, V2 and their raw ADC values V1, raw and V2, raw |
| --- | --- |

| r | Show the list of calibration constants that are stored within EEPROM |
| --- | --- |

| t | Show the contents of the trace circular buffer |
| --- | --- |

| ncells <integer> | Set the total number of cells within the battery pack Ncells, the value provided as an argument will be validated and stored in EEPROM |
| --- | --- |

| cfull <integer> | Set the battery design capacity Cfull in mAh, the value provided as an argument will be validated and stored in EEPROM |
| --- | --- |

| ichrg <integer> | Set the battery charging current Icharge, the value provided as an argument will be validated and stored in EEPROM |
| --- | --- |

| ifull <integer> | Set the end-of-charge current Ifull in mA, the value provided as an argument will be validated and stored in EEPROM |
| --- | --- |

| iut <index> <voltage> | Configure the state-of-charge lookup table (LUT). This command takes and index i=0,1,2,…,8 and the reference voltage li in mV as arguments. Each time this command is called, a new reference voltage value li is populated into the LUT and stored into EEPROM, more on this in the following section |
| --- | --- |

| rshunt <integer> | Set the shunt resistor value Rshunt in mΩ, the value provided as an argument will be validated and stored in EEPROM |
| --- | --- |

| cal <start¦stop¦v1¦v2> <mv> | The voltage calibration mode is entered by calling cal start and exited by calling cal stop.V1 is calibrated using cal v1 <mv>.V2 is calibrated using cal v2 <mv>.<mv> is the measured voltage level in millivolts. Please refer to the next section for more details about the calibration procedure. |
| --- | --- |

Table 4: command followed by one or two arguments separated

##### Calibration Procedure

This section provides an example on how to perform the first-time calibration of the Lithium-Ion battery charger using the CLI over the serial monitor.

The calibration values are stored into the Arduino’s electrically erasable programmable read-only memory (EEPROM). A cyclic redundancy check (CRC) checksum is appended to the configuration parameters set and stored into EEPROM as well. All configuration parameters are validated and out-of-range values are automatically replaced with the corresponding failsafe values.

The current example assumes a system consisting of Ncells= 4 connected in series having a design capacity of Cmax= 2500mAh charged using a current of Icharge = 1500mA.

Important:

- Do not connect the battery during the calibration procedure unless instructed otherwise.

- Ensure that the voltage calibration procedure has been properly executed and verified prior to attempting to connect a Lithium-Ion battery. It is mandatory to connect a good quality battery protection board between the charger and battery. Failing to observe these precautions may lead to permanent damage or even explosion of the Lithium-Ion cells.

###### Initial ranking

A first step, the initial configuration parameters need to be loaded into EEPROM by executing the command sequence as shown in figure 6 below:

Figure 6: command sequence

A confirmation message will be printed on the serial monitor following each value entry.

###### Voltage calibration

Having performed the above initial step, please proceed for calibrating the ADC readings for the voltages V1, V2 as shown below:

- Enter the Cal start command in the serial monitor, this will activate calibration mode. The Start Calibration message should appear on the serial monitor.

- Connect a constant voltage source of approximately 750 mV between the B- terminal and the power supply ground, and measure its exact value using a digital multimeter. Note that 750 mV corresponds to 1.5 A flowing through the shunt resistors R8 and R9.

- Enter the command cal v2 <value> into the serial monitor, where <value> is the value in mV of the voltage measured in the previous step. The value of the calibration constant V2,cal will be displayed upon the successful calibration of V2. If the calibration fails, the message Out of range will appear in the serial monitor.

- Connect a constant voltage source of approximately 16800 mV (4200 mV per cell) between the B+ terminal and the power supply ground, and measure its exact voltage using a digital multimeter.

- Enter the command cal v1 <value> into the serial monitor, where <value> is the value in mV of the voltage measured in the previous step. The value of the calibration constant V1,cal will be displayed upon the successful calibration of V1. If the calibration fails, the message Out of range will appear in the serial monitor.

- Verify the voltage calibration by applying a known voltage to each of B+ and B- (relative to 0 V), then enter the [.](dot) command and check the displayed values for V1 and V2 which must match the measured voltages at B+ and B- as closely as possible.

- Repeat steps 2, 3, 4, 5 and 6 until the voltage V readings are correct.

- Enter the command cal stop in order to exit the voltage calibration mode. The message Calibration stop should appear on the serial monitor.

###### Current calibration

Please proceed with calibrating the reading of the current I by following the steps below:

- Connect a discharged lithium-ion battery in series using a digital ampere meter (set to the 10 A range) to terminals B+ and B-.

- The message Charging should appear in the serial monitor and the measured current value should start to gradually increase until it reaches a maximum of approximately

1.5 A.

- Enter the [.] (dot) command and check the displayed value for I which must match the measured current as closely as possible.

- If the output of the [.] command is higher than the ampere meter reading: Increase the Rshunt value by 10 mΩ by calling the rshunt command.

- If the output of the [.] command is lower than the ampere meter reading: decrease the Rshunt value by 10 mΩ by calling the rshunt command.

- Repeat steps 3, 4, and 5 until the current I readings are correct.

##### Trace Buffer

A lithium-ion battery charger records events that occur during the charging process in a circular buffer within the available EEPROM space. The contents of the trace buffer are dumped using the t command. Here is a sample trace log output for a complete shipping cycle as shown in figure 7 below:

Figure 7: t command

Trace messages have the format <timestamp>: <event> <value>. While the timestamp counts the minutes that have passed since the beginning of the charging process. The following table shows the available events and their descriptions:

| Event | Description |
| --- | --- |
| * | Beginning of the charging cycle, indicates the maximum battery voltage Vmax in V |

| % | Initial charge state % |
| --- | --- |

| T | Maximum permissible charging time Tmax in minutes |
| --- | --- |

| C | Maximum permissible charging capacity Cmax in mAh |
| --- | --- |

| S | Safety charging in progress, Isafe is indicated in mA |
| --- | --- |

| I | Normal charge in progress, indicates Icharge in mA |
| --- | --- |

| v | Instantaneous battery voltage V=V1-V2 in mV |
| --- | --- |

| i | Instantaneous battery current I in mA |
| --- | --- |

| F | Battery full, indicates the end-of-charge condition (1 = Ifull reached, 2 = Cmax reached, 3 = Tmax reached) |
| --- | --- |

| t | Actual charging time T in minutes |
| --- | --- |

| c | Actual charged capacity C in mA |
| --- | --- |

| E | Error (1 = over-volt, 2 = under-volt, 3 = open-circuit, 99 = CRC fail) |
| --- | --- |

Table 5: Available events and descriptions

#### Download:

Below you can find GitHub download links for the Arduino firmware source code, Eagle schematic source files and bill of material. All of the source code is distributed under the GNU General Public License v3.0.

Please note that the current implementation uses the watchdog timer functionality which requires the customized Arduino bootloader found under the link below. For more details, please follow the installation instructions found within the README file on GitHub.

Customized Arduino Bootloader

Lithium-Ion Charger Firmware

Eagle Schematic Source Files

Bill of Material

## Agricultural Accessories

### Mechanics of Agricultural Acessories

#### Hitch

##### FreeCad Design

##### Mechanical Realization

##### Off-the-shelf device

### E/E of Agricultural Acessories

#### Working on controlling the crane (25/6/2023)

## Steps of work

### Assemblage the Tuk-Tuk	(10/1/2023)

### Installing a new structure for solar panels (25/1/2023)

After modifying it from the shape of the blue Tuk-Tuk’s structure.

### Small Adjustments (1/2/2023)

Raise the box level by 4 cm.

Changing the type of tire rubber

| Figure 1: Before | Figure 2: After |
| --- | --- |

### Modified basic mechanics: (8/2/2023)

#### Add 2 springs to the front wheel

#### Adjust the springs.

#### Changing the brake system (From the wire to the disc and the oil)

### Transfer the red tuk-tuk to Ras Nhache (25/2/2023)

### Trying the little blue dibble on the red tuk-tuk (12/3/2023)

### Work on the crane of the dibble for red and blue tuk-tuk’s (25/3/2023)

### Installing the big red Dibble on the blue Tuk-tuk. (15/4/2023)

### Installing the big red Dibble on the red Tuk-tuk. With modification to the crane base (17/4/2023)

### Testing the big Dibble (23/5/2023)

### Adjusting the crane base on the red tuktuk (30/5/202)

### Paint the Tuk-tuk red (1/6/2023)

### Testing the Tuk-tuk (6/6/2023)

### Receiving the store next to the center and transporting the Tuk-tuk to it from Ras Nhach.(10/6/2023)

### Design and installation of an irrigation system on a red Tuk-tuk. With some modifications from the old system in the blue Tuk-tuk (22/6/2023)

### Grass shredder installation.	(3/7/2023)

### Installing a 12V water pump on the red Tuk-tuk (7/7/2023)

### Irrigation test (10/7/2023)

### 20- Installing two front tires instead of one tire on the red tuk-tuk. 				(16/8/2023)

### Testing and Marketing the red Tuk-tuk (20/8/2023)

### The new Tuktuk's front budget is painted red (31/8/2023)

### Used the red tuk-tuk by AECENAR team(Power plan) (20/8/2023	to present)

TBD : 3D and 2D designs fort he following parts :

Motor
The heart and soul of your vehicle is the internal electrical motor

Transmission
The transmission is a gearbox filled with gears and gear trains that makes effective use of the motor’s torque to change the gears and power the vehicle.

Battery
The battery delivers the electricity needed to run your vehicle’s electrical components. Without a battery, your car won’t work.

Charger
Part of the electrical system, the charger charges the battery.

Front Axle
Part of the suspension system, the front axle is where the front wheel hubs are attached.

Front Steering and Suspension
Helps improve the ride and handling of the vehicle. Though systems vary in makeup, they typically include shocks/struts, ball joints, tie rod ends, rack and pinion steering system and idler/pitman arms.

Brakes
Found on all four wheels, your brakes are one of the most important safety systems on your vehicle. Disc brakes can be found on the front and back wheels and feature brake pads and calipers. Drum brakes with brake shoes and wheel cylinders may be found on the back wheels of some vehicles.

Rear Axle
Key part of the suspension system to which the rear wheels are mounted.

Rear Suspension
As with the front suspension, the rear suspension contributes to the handling and ride quality of the vehicle. Systems can vary, but they usually are made up of shocks, coil springs, ball joints, control arms and CV joints

# To be improved

## Power of DC motor

Gasoline Tuk-Tuks on market have about 7-8 kW

The NLAS E-Transporter should have about 3-5 kW

## More own parts (except of chassis with number)

# Proposal for Investment in Sustainable Electric Car Project

From: MMJZ and ZM , NLAS Local Factory (100 vehicles per year), Businessplan for Investor Zakariyya (UBSA), March 2024

## Executive Summary

We are pleased to present our proposal for your consideration to invest in our sustainable electric car project. This initiative aims to introduce cutting-edge electric vehicles into the market, addressing the increasing demand for eco-friendly transportation solutions. There are 2 different models presented in our proposal namely Cargo and Passenger Electric Vehicle

The initial stage focuses on a cargo vehicle. This approach allows for a strategic market entry, establishing reliability and functionality. The subsequent phase will transition to passenger transport.

## Project Overview

Our project involves the development of electric vehicles powered by solar energy and lithium-ion batteries (further detailed description is clearly defined in Annex I).

Based on our market research and following our latest agricultural vehicle prototype (See Figure 1) that has garnered positive feedback from potential customers who expressed enthusiasm for its use, we realized that there is a keen interest in cargo transport rather than agricultural applications, indicating a shift in market demand towards versatile electric vehicles.

The target market for electric vehicles is substantial, with potential growth projections indicating a lucrative opportunity. Regulatory trends favoring sustainable transportation further enhance the market landscape.

Our study has shown that electric vehicles do have advantages over traditional cars especially in maintenance and daily operation (Fuel consumption). According to our business plan analysis (detailed in ANNEX II), electric cars save around 75% on Maintenance and 67% on normal operations.

Figure 1: Cargo Electric Car Sample

## Business plan

We are looking to setup a factory to deliver the forecasted cargo and passenger EV. A land is required for this matter (approx. 4000m2) where we need to install a metal structure container that serves the factory vicinity and production plant.

We are forecasting to produce 50 cars each year (25 per each model)

### Partnership Duration

We are forecasting a 3-year business plan. The project duration is extendable as the need arise and based on the performance of the first 3 years

### Business Model Partnership:

We propose a profit-sharing model as follows:

- First phase (cargo vehicle), with a distribution of 60% for NLAS and 40% for the investor. (R&D already covered by NLAS)

- Second phase (passenger vehicle), the profit-sharing ratio becomes 50% for NLAS and 50% for the investor.

### Pricing Strategy:

Our pricing strategy is crafted to ensure competitiveness in the electric vehicle market, taking into account the unique features of our electric cars. Given the limited presence of such vehicles in the region or the absence of strong competitors, our strategic position allows us to address a market need.

Considering the niche nature of the market, we recognize the potential for establishing favorable pricing. The absence of strong competition positions us as pioneers, and we aim to capitalize on this advantage.

We are looking forward to put initial target market price in as follows:

- Cargo EV: 3500$ (Lithium as optional with additional 500$)

- Passenger EV: 5500$ (Lithium as optional with additional 500$)

Details on pricing strategy will be explained under financial sections

### Responsibility Matrix

NLAS will be responsible for R&D, operations, and implementation of the project. Should the investor like to get involved, it is welcomed based on a defined criteria and strategy in order to clarify all roles. As for the marketing and sales, the investor public relation and experience in local market will be added value in the sale channels and business development of the company (Detailed RM in Annex III)

Furthermore, the investor will play a pivotal role in sales and marketing functions. The investor's involvement will extend to overseeing sales strategies, marketing campaigns, and establishing partnerships. This collaborative approach ensures that the investor actively contributes to the success of our pricing strategy in a market where our presence is particularly vital.

As for the funds required for the execution of the project, it is the responsibility of the investor to make sure to avail the required cash at the startup of the project upfront

## 4. Financials

### Budget estimation

The following breakdown of cost estimation to derive the budget for delivering 1 cargo and passenger EV

#### Factory Setup

| Description | Unit | Price | Duration | Total |
| --- | --- | --- | --- | --- |
| Land lease | Yearly | $ 15,000 | 3 | $ 45,000 |
| factory garage | Lum | $ 25,000 | - | $ 25,000 |
| Power BKP | Lum | $ 5,000 | - | $ 5,000 |
|  |  |  |  | $ 75,000 |

#### HR, Operations and integrations

| Desc | Y1 | Y2 | Y3 |
| --- | --- | --- | --- |
| CARGO |  |  |  |
| operation/int | $ 6,000 | $ 6,000 | $ 6,000 |
| PASSENGER |  |  |  |
| operation/int | $ 15,000.00 | $ 15,000.00 | $ 15,000.00 |

#### R&D

| Desc | UNIT | Price | Y1 | Y2 | Y3 |
| --- | --- | --- | --- | --- | --- |
| R&D Cargo | LS | $ 5,000 | $ 5,000 | $ 5,000 | $ 5,000 |
| R&D Passenger | LS | $ 6,000 | $ 6,000 | $ 6,000 | $ 6,000 |

The R&D and Operation cost are leveraged over a period of 3 years and based on production of 25 EV per year for each type. We can improve the cost of EV production by increasing the capacity of production which requires more capital. Depend on the investor strategy, we can work another exercise for targeting 100 EV per year thus improve margin cost by 23% per EV

#### Summary

Accordingly Budgetary estimation for the cost of:

- CARGO EV :

| Desc | Yeary | Per car |
| --- | --- | --- |
| Qty | 25 |  |
| Factory setup | $ 12,500 | $ 500 |
| operation/int | $ 6,000 | $ 240 |
| R&D | $ 5,000 | $ 200 |
| Material | $ 37,500 | $ 1,500 |
| TOTAL |  | $ 2,440 |

PASSENGER EV

| Desc |  | Y1 | Y2 | Y3 |
| --- | --- | --- | --- | --- |
| Qty |  | 25 | 25 | 25 |
| PASSENGER Cost |  | $ 96,000 | $ 96,000 | $ 96,000 |
| PASSENGER Revenue |  | $ 137,500 | $ 137,500 | $ 137,500 |
| Profit |  | $ 41,500 | $ 41,500 | $ 41,500 |
| Capital Invest |  | $ 96,000 | $ 96,000 | $ 96,000 |
| Investor Profit share | 50% |  | $ 14,500 | $ 14,500 |
| NLAS Profit Share | 50% |  | $ 14,500 | $ 14,500 |

In Summary the investor is required to provide capital invest of 56k$ for CARGO and/or 96k$ for PASSENGER EV with respective profit shares of 40/60 for Cargo (since R&D already covered) and 50/50 for PASSENGER

## 5. Conclusion and Call to Action

In conclusion, our electric car project presents an exciting opportunity to revolutionize sustainable transportation in the region. With innovative features such as solar energy integration, lithium-ion batteries, and a strategic phased approach from cargo to passenger transport, our project is poised to make a significant impact.

By seeking your investment, we aim to accelerate the development and deployment of our electric cars, addressing the market's growing need for eco-friendly, reliable transportation solutions. Your expertise in marketing and sales, coupled with your strategic vision, will play a crucial role in the success of our venture.

We invite you to further discuss the details of our proposal and explore how your collaboration can propel our electric car project to new heights. Together, let's embark on a journey to shape the future of sustainable mobility.

6. Investment Terms:

- The partnership continues as long as the investor fulfills their duties and responsibilities.

- The investor is a production partner only and is not allowed to use or sell the sources, layouts, study during or after the contract.

We look forward to the opportunity to meet and discuss the exciting possibilities that lie ahead.

Thank you for considering our proposal.

Dr. Samir Mourad, CEO

## ANNEX I : Product Description

### Vehicle Specification

- General:

  - Motor Power: 1500 W

  - Battery: 60 V, 50 Ah

  - Max Speed: 60 km/h

  - Distance on a single charge: 60 km

  - Charging time: on power: 6 hours, on solar: 4 hours

    - Capacity: Cargo version: Max: 600 kg

    - Passenger version: 4 passengers (total: 500 kg)

- Optional Feature:

  - Solar integration

  - Lithium battery

### Solar Integration:

Solar energy, captured by photovoltaic cells on the vehicle's roof, directly supplements the vehicle's power source, typically batteries. This additional energy input extends the battery life and enhances overall vehicle efficiency. By harnessing solar power, the vehicle's reliance on external charging is reduced, leading to increased sustainability and improved range.

### Lithium Battery:

Lithium-ion batteries, commonly used in electric vehicles for their high energy density and long cycle life, are available as an optional feature due to their additional cost. Advances in technology aim to boost energy density, reduce charging times, and enhance safety. Despite their benefits, the optional nature of lithium-ion batteries acknowledges the additional expense they incur, allowing consumers to make choices based on their preferences and budget.

### Safety Measures:

Our extensive experience in this field will be leveraged through our involvement in various projects with the majority of German companies (One of our recent collaborations was with JOPP). This car will design and manufacture to the high practice safety and quality standards.

We are committed to providing a safe and reliable transportation environment, considering safety as a non-negotiable aspect in the design and production of our electric vehicles.

### Customization Options:

We have the capability and willingness to make changes and customizations to the vehicle based on specific customer needs. Whether it involves modifications to meet particular market requirements, additional features, or design adjustments, our team is flexible and responsive to ensure that the electric car aligns perfectly with the unique preferences and demands of our customers.

## ANNEX II : Market study - Maintenance and operations comparison and cost saving

### Maintenance Expenses:

Traditional Cars: Internal combustion engine vehicles have a complex mechanical structure that often requires frequent maintenance, such as oil changes, exhaust system repairs, and transmission adjustments.

Electric Cars: Electric vehicles have fewer moving parts, leading to significantly lower maintenance requirements. The absence of traditional components like oil filters, spark plugs, and complex exhaust systems translates to reduced servicing needs.

### Fuel Expenses

Traditional Cars: Reliance on gasoline or diesel can be a significant ongoing expense, especially with fluctuating fuel prices.

Electric Cars: Charging an electric vehicle is generally more cost-effective than filling up a traditional vehicle with gas.

|  | $ Cost per year |  |  | Saving | % |
| --- | --- | --- | --- | --- | --- |
|  | Traditional Car | Electric Car |  |  |  |
| Maintenance | 800 | 200 | 600$ |  | 75% |
| Fuel/Charge | 1500 | 500 | 1000$ |  | 67% |

By transitioning to our electric car, users can potentially save a total of $1,600 per year. These estimations provide a tangible demonstration of the economic advantages and long-term cost savings associated with adopting our sustainable electric vehicle.

## ANNEX III : Responsibility Matrix table

| Task | Investor | NLAS |
| --- | --- | --- |
| Research and Design | - | R |
| Budget Estimation | A | R |
| Component Procurement | - | R |
| Production | - | R |
| Testing and Quality Assurance | A | R |
| Marketing and Promotion | R | - |
| After Sales | A | R |
| Capital Funds | R | - |

Legend:

R = Responsible (Person or entity responsible for completing the task)

A = Accountable (Person or entity ultimately accountable for the task's success)

- = Not Applicable

## Annex IV : Phase Budget Estimation

### Phase 1: Cargo Vehicle

1. Research and Design:

- R&D Costs: None already done by NLAS

2. Vehicle Production:

- Body Factory Cost: 800 $/vehicle

- Materials Procurement: 400 $/vehicle

- Solar system: 300 $/vehicle (Optional)

- Lithium battery: 400 $/vehicle (Optional)

- Assembly Cost: 200 $/vehicle

Total Vehicle production cost: 2100 $

### Phase 2: Passenger Vehicle

1. Design Changes:

- Duration: 3 months

- Engineers Costs: 2000 $/month

Total R&D estimated cost: 6000 $

2. Vehicle Production:

- Body Factory cost: 800 $/vehicle

- Vehicle Interior cost: 500 $/vehicle

- Materials Procurement: 500 $/vehicle

- Solar system: 300 $/vehicle (Optional)

- Lithium battery: 400 $/vehicle (Optional)

- Assembly Cost: 300 $/vehicle

Total Vehicle production cost: 2800 $

3. Additional Tasks for Passenger Vehicle:

- Safety Standard Compliance: 2000 $ consulting cost

- Formal Registration and Agreements: 1000 $

- Testing and Enhancements: 1000 $/month

Total Estimated Cost for Additional Tasks: 4000$

*Note: all costs and prices are estimated and subject to change based on current market conditions.

### ANNEX V : Ready to Market Time Plan

#### Phase 1: Cargo Vehicle

##### 1. Research and Design:

- Duration: None

##### 2. Cargo Vehicle Production:

Chassis Production:

- Factory Agreement for body: 1 month

- Materials Procurement quotation: 1/2 month

- Assembly Duration: 1/2 month

Grand Total Time for Cargo vehicles: 2 months

#### Phase 2: Passenger Vehicle

##### 1. Design Changes:

- Duration: 3 months

##### 2. Vehicle Production:

Chassis Production:

- Duration: 1 month

##### 3. Car Interior Upholstery:

- Duration: 1 month

##### 4. Additional Tasks for Passenger Vehicle:

Safety Standard Compliance, Formal Registration, and Agreements, Testing, and Enhancements:

- Duration: 2 months

Grand Total Time for passengers vehicle: 7 months

# Production Facility

## Production Line

Development: Ras Maska, Integration: Ras Nhache, Sales: Tripoli

Author: Anas Hamzeh

Solidworks STEP file shows the production line 3D model.

### Costs Excel table:

### Production Line (PL) development:

After completing the first version of the production line, we identified that some stations will not be crucial during the initial stages of production since the main chassis parts will be imported.

The production line will be constructed within a 40-foot (approximately 12 meters) container, divided into three stations, each 4 meters long. The first station will be designated for bodywork, where weldments and painting will be performed if necessary. The second and third stations will be used for all assembly tasks (including electrical, electronics, and mechanical assembly), with quality tests conducted throughout the assembly operations. Given the limited space, a buffer storage area is essential to hold the stock of parts required for the production line.

### Manufactoring Manual & Production Timeline

#### NLAS E-Transporter Prototype BOM

| NLAS E-Transporter23 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Using red Transportation Tuk-Tuk for delivery issues |  |  |  |  |  |  |
| 2. Production Business Plan: |  |  |  |  |  |  |
| https://aecenar.com/index.php/downloads/send/7-association-for-alternative-energy-research-vaef/1579-nlas-business-plan-2024-docx |  |  |  |  |  |  |
| Products |  |  |  |  |  |  |
|  | Main Materials |  | Used Systems | Develop- ment | Develop- ment | SOP |
| Transportation E-Tuktuk |  |  |  | Oct 22-Oct24 | Oct 22-Oct24 | Nov 24 |
| NLAS E-Transporter23 Parts |  |  |  |  |  |  |
| Part | quantity | picture | description | Supplier1 | Supplier2 | Price per item |
| Basic (originally Tuk-Tuk) parts except of chassis |  |  |  |  |  | $200 |
| Front sides flashers | 1 |  | the two are connected to each other |  |  |  |
| Rear flashers | 2 |  |  |  |  |  |
| differartial regulation axe | 2 |  |  |  |  |  |
| rear brake axe | 3 |  |  |  |  |  |
| flasher coil 60V | 1 |  |  |  |  |  |
| pins for rear dampers | 12 |  |  |  |  |  |
| digital 60V dashboard | 1 |  |  |  |  |  |
| T shape front suspension axe | 1 |  |  |  |  |  |
| 12 inch rear rims | 2 |  | without standard tires |  |  |  |
| start switch | 1 |  |  |  |  |  |
| rear differatial axe | 1 |  | with gear and fising pins |  |  |  |
| front steering wheel bearing | 1 |  |  |  |  |  |
| control circuit for all electric parts | 1 |  |  |  |  |  |
| Gear shifter rod | 1 |  |  |  |  |  |
| brake sensor | 1 |  |  |  |  |  |
| 6 pins connector box | 1 |  | for power distribution |  |  |  |
| steering wheels switches and buttons | 1 set |  |  |  |  |  |
| supports for rear damper | 2 |  |  |  |  |  |
| controller 1500w 72v BRSH-60 | 1 |  |  |  |  |  |
| steering wheel | 1 |  |  |  |  |  |
| Gear shifter handle | 1 |  |  |  |  |  |
| hand brake handle | 1 |  |  |  |  |  |
| speed controller FR-3S | 1 |  |  |  |  |  |
| rear dampers | 2 |  |  |  |  |  |
| electric motor 1500w 72v | 1 |  |  |  |  |  |
| Solar panels parts |  |  |  |  |  | $400 |
| MPPT | 1 |  |  |  |  |  |
| solar panels 100w | 5 |  |  |  |  |  |
| solar system connectror and wires | 1 set |  |  |  |  |  |
| front suspension parts |  |  |  |  |  | $400 |
| Front shock absorber | 2 |  | 350mm for 200cc ATV |  |  |  |
| Upper swing arm | 2 |  |  |  |  |  |
| Lower swing arm | 2 |  |  |  |  |  |
| steering strut knuckle spindle and wheel hub | 2 |  |  |  |  |  |
| Bolts | 4 |  |  |  |  |  |
| chassis and body |  |  |  |  |  | $1.000 |
| long chassis with front suspesion chassis | 1 |  |  |  |  |  |
| battery container and driver seat | 1 |  |  |  |  |  |
| cabin | 1 |  |  |  |  |  |
| solar panels stand | 1 |  |  |  |  |  |
| Batteries |  |  |  |  |  |  |
| Lithium Bat Block (70d delivery) | 1 |  | I_max=40A, 100Ah, 20 cells (45$ per cell), ECU: 100$, structure: 50$ |  |  | $1.050 |
| Integration (4 d mech, 2d E/E) |  |  |  |  |  | $300 |
|  |  |  |  |  | Sum | $3.350 |

## Building of Facility in Ras Nhache

## Hangar Facility in Ras Nhache

## Costs

### Surface leveling of ground

About 2500$

### Radier (about 250qm)

About 18000$

## Walls and Ceiling Estimation

# Mafrak – Ras Nhache line

## Business Specifications

Price: 50.000 L.L. (about 0,5 cent)

Only for females

Stations: Masjid or Madrasa

## Design of Persons Transport

Transporter: 2 opposite banks for 2x2 persons

Reference

NLAS Annual Report 2022

https://aecenar.com/index.php/companies/nl-automotive-systems-nlas/e-tuktuk/e-tuktuk-mechanical-realization

https://aecenar.com/index.php/downloads/send/7-association-for-alternative-energy-research-vaef/263-lithiumbat-spec

From Build a Lithium-Ion Battery Charger on Arduino | μF (microfarad.de) (https://www.microfarad.de/li-charger/)